| Item Name | Units | Data Type | Size in bits | Notes |
| --- | --- | --- | --- | --- |
| timetag | gps | double | 64 | Time of event in gps |
| level2Version |  | byte | 8 | Version of level 2 file |
| channel |  | byte | 8 | ID of the channel |
| orbitNum |  | int | 32 | Orbit number during which the event occurred. |
| fileId |  | int | 32 |  |
| productId |  | smallint | 16 |  |
| level2Version |  | byte | 8 | Version of level 1 file |
| charge | Number of electrons | double | 64 | Charge generated from each event |
| qualityFlag |  | int | 32 | Integer representing various two-state quality flags. |
| mass | grams | double | 64 | Mass of a particle that would cause this event. |
| msigPlus | grams | double | 64 | Plus sigma for mass generated |
| msigMinus | grams | double | 64 | Minus sigma for mass generated |
| threshold | grams | double | 64 | Threshold in grams |
| latitude | degrees | double | 64 |  |
| longitude | degrees | double | 64 |  |
| altitude | km | double | 64 | Altitude from earth center |
| glat | degrees | double | 64 | CDE defined coordinate system with 0-360 degrees of latitude |

Notes:

- All events are turned into charge regardless of whether or not they are suspected as noise events.  This will be dealt with later.

- This data is created from the calibration curves established on the ground.  These curves are a function of temperature, channel number, and DN.