| SOFIE Level 1 netCDF file Description |  |
| --- | --- |
| SOFIE_Level_1_netcdf_file_description_v1.0 | Date: February 8, 2008 |

| Revisions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rev | Description of Change | By | Approved | Date |
| 1.0 | Draft/Initial version | E. Thompson |  | 2/8/2008 |

| # Name | # Long name | Valid min | Valid max | Missing value | Fill value | Units/description |
| --- | --- | --- | --- | --- | --- | --- |
| Int event(event) | "Level 1 Event Number for this Day" | "0" | "31" | "0" | "0" | "Number of events in day" |
| Int orbit_number(event) | "Orbit Number" | "1" | "10000" | "0" | "0" | "Number of orbits since launch" |
| Int sunrise_sunset_flag(event) | "Sunrise/Sunset Flag" | "0" | "1" | "2" | "2" | "0=sunrise, 1=sunset" |
| Int event_start_time(event) | "Event Start Time" | ------- | ------- | "-1” | "-1" | "seconds since the unix epoch" |
| Int event_end_time(event) | "Event End Time" | ------- | ------- | "-1" | "-1" | "seconds since the unix epoch" |
| double time(event, time) | "time during measurement" | "0.f" | "3000.00" | "-1.e+24f" | "-1.e+24f" | "seconds since start of the event" |
| double scan_angle(event, scan_angle) | "Scan Angle for SLDC" | "-0.0012f" | "0.010f" | "-1.e+24f” | "-1.e+24f" | "radians" |
| CB double ViewingAngle(event, time) | "Ephemeris Viewing Angle" | "0.00" | "1.f" | "-1.e+24f" | "-1.e+24f” | "radians" |
| CB double TanPointAlt(event, time) | "Tangent Point Altitude" | "-100.f" | "400.f" | "-1.e+24f" | "-1.e+24f" | "km" |
| CB double TanPointLat(event, time) | "Tangent Point Latitude" | "-90.f" | "90.f" | "-1.e+24f" | "-1.e+24f" | "degrees" |
| CB double TanPointLon(event, time) | "Tanget Point Longitude" | "0.f" | "360.f" | "-1.e+24f" | "-1.e+24f" | "degrees" |
| CB double Atmospheric_Doppler_Velocity(event, time) | "Atmospheric Doppler Velocity” | "-10.f" | "10.f" | "-1.e+24f" | "-1.e+24f" | "km/s" |
| double Signal(event, detector_no, time) | "Detector Signals" | "0.f" | "40000f" | "-1.e+24f" | "-1.e+24f" | "Counts" |
| double Diff_Signal(event, diff_channels, time) | "Difference Signals" ; |  |  | "-1.e+24f" | "-1.e+24f" | "Counts" |
| double Signal_Offset(event, detector_no) | "dark current offset" |  |  | "-1.e+24f” | "-1.e+24f” | "Counts" |
| double Signal_Drift(event, detector_no) | "Signal Drift Parameter" |  |  | "-1.e+24f" | "-1.e+24f" | "Counts/sec" |
| double Signal_Confidence(event, detector_no) | "Signal Confidence (Std. Dev. Of signal determined during drift correction”" |  |  | "-1.e+24f" | "-1.e+24f" | "Counts/sec" |
| double SLDC(event, detector_no, scan_angle) | "Solar Limb Darkening Curve" | "0.0f" | "1.0.f" | "-1.e+24f" | "-1.e+24f" | "Normalized to 1.0" |
| double Refraction_Angle(event, time) | "Refraction angles from FPA (solar extent at 700nm)" | "0.f" | "90.f" | "-1.e+24f" | "-1.e+24f" | "radians" |
| double merged_altitude(event, merged_altitude) | "Altitudes for merged NCEP and MSIS profile" | "0.f" | "150.f" | "-1.e+24f" | "-1.e+24f" | "km" |
| double merged_Pressure(event, merged_altitude) | "Pressures from merged NCEP and MSIS profile" | "1.e-5f" | "11000.f" | "-1.e+24f" | "-1.e+24f” | "mbar" |
| double merged_Temperature(event, merged_altitude) | "Temperatures from merged NCEP and MSIS profile" | "0.f" | "350.f" | "-1.e+24f" | "-1.e+24f" | "Kelvin" |
| Int reg_detectors(event, reg_detectors) | "Detectors used for registration" | "1" | "16" | "-1" | "-1" | "N/A" |
| CB double Alt_Reg_Ref(event, reg_detectors) | "Altitude where altitude registration performed" | "-10.f" | "10.f" | "-1.e+24f" | "-1.e+24f" | "Counts" |
| CB double Alt_Reg_Shift(event, reg_detectors) | "Altitude Shift caused by registration" | "-10.f" | "10.f" | "-1.e+24f" | "-1.e+24” | "km" |

Where the dimensions are:

dimensions:

event = UNLIMITED ; // (26 currently)

time = 3227 ;

scan_angle = 1000 ;

detector_no = 16 ;

diff_channels = 8 ;

reg_detectors = 3 ;

merged_altitude = 100 ;

refraction_alt = 100

and the global attributes are:

// global attributes:

:Title = "SOFIE Level1 Pre-Launch" ;

:DP_Type = "Level1 Pre-Launch" ;

:Source = "SOFIE DPC" ;

:Mission = "AIM" ;

:DP_Version = "0.01" ;

:PF_Version = "0.01" ;

:SW_Version = "0.01" ;

:SW_Name = "Sofie Level1" ;

:Calib_Version = "0.01" ;

:Description = "Sofie Level1 Data Product" ;

:History = "Pre-Launch" ;

:Gen_Date = "2007-04-05" ;