Reference “The Sofie Electrical and Software Interface Control Document SDL/04-040 rev B” for additional information

Detector Samples:

| DB Field | Units | Description | Related Equations |
| --- | --- | --- | --- |
| SAMPLE_ID | bigint(20) | pk |  |
| PACKET_ID | bigint(20) | Identifier to relate science data |  |
| SAMPLE_NUMBER | tinyint(4) | 1 of 7 samples. (7 samples of info per packet) |  |
| REPROCESSING_COUNT | int(11) | Indicates the number of times the data has been processed |  |
| VERSION_ID | varchar(8) | Version of Level0 Software used to process data |  |
| DETECTOR_TIME_STAMP | datetime | Packet time value in “2007-02-05 05:10:13” format |  |
| SAMPLE_MICROSECONDS | int(11) | Packet time value in microseconds |  |
| DETECTOR_1 | int(11) | Counts for detector channel 1 O3 (s) | (#THIS# / 32767) * 3 |
| DETECTOR_2 | int(11) | C Counts for detector channel 2 O3 (w) |  |
| DETECTOR_3 | int(11) | Counts for detector channel 3 PMC |  |
| DETECTOR_4 | int(11) | Counts for detector channel 4 PMC |  |
| DETECTOR_5 | int(11) | Counts for detector channel 5 H2O (w) |  |
| DETECTOR_6 | int(11) | Counts for detector channel 6 H2O (s) |  |
| DETECTOR_7 | int(11) | Counts for detector channel 7 CO2 (s) |  |
| DETECTOR_8 | int(11) | Counts for detector channel 8 CO2 (w) |  |
| DETECTOR_9 | int(11) | Counts for detector channel 9 PMC |  |
| DETECTOR_10 | int(11) | Counts for detector channel 10 PMC |  |
| DETECTOR_11 | int(11) | Counts for detector channel 11 CH4 (s) |  |
| DETECTOR_12 | int(11) | Counts for detector channel 12 CH4 (w) |  |
| DETECTOR_13 | int(11) | Counts for detector channel 13 CO2 (s) |  |
| DETECTOR_14 | int(11) | Counts for detector channel 14 CO2 (w) |  |
| DETECTOR_15 | int(11) | Counts for detector channel 15 NO (w) |  |
| DETECTOR_16 | int(11) | Counts for detector channel 16 NO (s |  |
| DIFFERENCE_1_2 | int(11) | Difference between detector signals 2-1 |  |
| DIFFERENCE_3_4 | int(11) | Difference between detector signals 4-3 |  |
| DIFFERENCE_5_6 | int(11) | Difference between detector signals 6-5 |  |
| DIFFERENCE_7_8 | int(11) | Difference between detector signals 8-7 |  |
| DIFFERENCE_9_10 | int(11) | Difference between detector signals 10-9 |  |
| DIFFERENCE_11_12 | int(11) | Difference between detector signals 12-11 |  |
| DIFFERENCE_13_14 | int(11) | Difference between detector signals 14-13 |  |
| DIFFERENCE_15_16 | int(11) | Difference between detector signals 16-15 |  |

Diagnostic_Packets

| DB Field | Units | Description |
| --- | --- | --- |
| PACKET_ID | bigint(20) | pk |
| PACKET_DATETIME | datetime | Packet time value in “2007-02-05 05:10:13” format |
| PACKET_MILLISECONDS | smallint(6) | Milliseconds value extracted from the packet header |
| PACKET_APID | smallint(6) | APID of packet being currently processed (0x185) |
| PACKET_CRC | int(11) | Packet CRC32 checksum |
| PACKET_IS_CLEAN | tinyint(4) | Currently, this value is always “1” |
| PACKET_SOURCE_SEQUENCE_COUNT | smallint(5) | CCSDS Packet Source Sequence Count |
| PACKET_LENGTH | smallint(5) | CCSDS Packet Length |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |
| VERSION_ID | varchar(8) | Version of Level0 Software used to process data |
| DIAGNOSTIC_PACKET_IDENTIFIER | int(10) | Specifies the type of diagnostic packet (EEPROM, SRAM, instruction SRAM, etc.) |
| DIAGNOSTIC_PACKET_BUFFER | blob | Contains the binary information from the data section of the packet. |
| DIAGNOSTIC_PACKET_BUFFER_LENGTH | int(10) | Length of the binary Diagnostic Packet Buffer. |
| PACKET_SOURCE_SEQUENCE_FLAGS | int(11) | CCSDS flags generated from the instrument about the source sequence counter. |

Event_Ancillary:

| DB Field | Units | Description |
| --- | --- | --- |
| EVENT_ID | bigint(20) | Pk |
| EVENT_TYPE | varchar(32) | Sunrise or sunset event |
| ORBIT_NUMBER | int(11) | orbit number that corresponds to the event start time, defined in the orbit_numbers.txt file for level0b code |
| BEGIN_TIME | datetime | Start time for the event |
| END_TIME | datetime | Ending time of the event |
| TABLE_NUMBER | int(11) | Which event table is in use |
| EVENT_NUMBER | int(5) | Current CommandHistory.txt table event number |
| 30_KM_AVERAGE_LAT | int(11) | Currently, both values are NULL |
| 30_KM_AVERAGE_LON | int(11) |  |
| CAL_START_TIME | datetime | Beginning time for calibration |
| CAL_END_TIME | datetime | Ending time for calibration |
| CAL_TYPE | varchar(32) | Curent event calibration type (the current event table number) |
| OCC_START_TIME | datetime | Beginning time for occultation |
| OCC_END_TIME | datetime | Ending time for occultation |
| AVERAGE_BETA | double | Currently, this value is always NULL |
| SPACECRAFT_LAT | int(11) | Latitude of the spacecraft |
| SPACECRAFT_LON | int(11) | Longitude of the spacecraft |
| SPACECRAFT_ALT | int(11) | Altitude of the spacecraft |
| IS_PREDICTED_EVENT | tinyint(4) | Currently, this value is always “0” |
| DROPPED_PACKETS | int(11) | Total number of dropped packets |
| HK_DATA_QUALITY | int(5) | Number of dropped packets in housekeeping packet |
| DIAG_DATA_QUALITY | int(5) | Number of dropped packets in diagnnostic packet |
| DC_DATA_QUALITY | int(5) | Number of dropped packets in dc packet |
| SYS_DATA_QUALITY | int(5) | Number of dropped packets in system packet |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |

EVENT_TAGS:

| DB Field | Units | Description |
| --- | --- | --- |
| EVT_TAG_ID | Int(10) | pk |
| TIME | datetime | Event Time |
| TAG_TYPE | Varchar(15) | 1 of 4 values: OCC, CAL, EVENT, or BALANCE |
| PARAM1 | Varchar(5) | For OCC, CAL, EVENT tags, this value will be either BEGIN or END. For the BALANCE tag, this value will be the Detector Number |
| PARAM2 | Varchar(5) | Only applicable for the BALANCE tag. Value will be the ATTENUATOR SETTING in percentage of full signal. (0 – 1 Where 0 is full attenuation with no signal and 1 is no attenuation with full signal. |
| VERSION_ID | Varchar(8) | Level0b Software version used to process data |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |

Housekeeping _Packets:

| DB Field | Units | Description | Related Equations |  |
| --- | --- | --- | --- | --- |
| PACKET_ID | bigint(20) | Pk |  |  |
| PACKET_DATETIME | datetime | Packet time value in “2007-02-05 05:10:13” format |  |  |
| PACKET_MILLISECONDS | smallint(6) | Milliseconds value extracted from the packet header |  |  |
| PACKET_APID | smallint(6) | APID of packet being currently processed (0x180) |  |  |
| PACKET_CRC | int(11) | Packet CRC32 checksum |  |  |
| PACKET_IS_CLEAN | tinyint(4 | Currently, this value is always “1” |  |  |
| PACKET_SOURCE_SEQUENCE_COUNT | smallint(5) | CCSDS Packet Source Sequence Count |  |  |
| PACKET_LENGTH | smallint(5) | CCSDS Packet Length |  |  |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |  |  |
| VERSION_ID | varchar(8) | Version of Level0 Software used to process data |  |  |
| APERTURE_HOUSING_TEMPERATURE | int(11) | (ext) temperature sensor, Aperture housing | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_1_REFERENCE_RESISTOR_1380 | int(11) | (int) Reference resistor, 1.38 KOhms = 373 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_1_REFERENCE_RESISTOR_200 | int(11) | (int) Reference resistor, 200 Ohms = 77 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_2_REFERENCE_RESISTOR_1380 | int(11) | (int) Reference resistor, 1.38 KOhms = 373 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_2_REFERENCE_RESISTOR_200 | int(11) | (int) Reference resistor, 200 Ohms = 77 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_3_REFERENCE_RESISTOR_1380 | int(11) | (int) Reference resistor, 1.38 KOhms = 373 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_3_REFERENCE_RESISTOR_200 | int(11) | (int) Reference resistor, 200 Ohms = 77 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_4_REFERENCE_RESISTOR_1380 | int(11) | (int) Reference resistor, 1.38 KOhms = 373 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHANNEL_4_REFERENCE_RESISTOR_200 | int(11) | (int) Reference resistor, 200 Ohms = 77 Kelvin | 0.01171824341 * #THIS# - 256.0333 |  |
| CHOPPER_HEALTH_LEFT_CHANNEL_TEMPERATURE | int(11) | Chopper health left channel | 0.00004577637 * #THIS# |  |
| CHOPPER_HEALTH_RIGHT_CHANNEL_TEMPERATURE | int(11) | Chopper health right channel |  |  |
| CHOPPER_PCB_TEMPERATURE | int(11) | (int) temperature sensor, Chopper PCB | 0.01171824341 * #THIS# - 256.0333 |  |
| COMMAND_AND_DATA_HANDLING_PCB_TEMPERATURE | int(11) | (int) temperature sensor, C&DH PCB | 0.01171824341 * #THIS# - 256.0333 |  |
| CSM_NEAR/FAR_FILTER_RETAINER_TEMPERATURE | int(11) | (ext) temperature sensor, CSM near/far filter retainer | 0.01171824341 * #THIS# - 256.0333 |  |
| CSM_NEAR_OPTICS_HOUSING_TEMPERATURE | int(11) | (ext) temperature sensor, CSM near optics housing #1 |  |  |
| DETECTOR_10_TEMPERATURE | int(11) | Detector 10 Temperature | (-5.58394e-34 * (#THIS# ^ 7)) + (5.1701e-28 * (#THIS# ^ 6)) + (-1.91918e-22 * (#THIS# ^ 5)) + (3.65373e-17 * (#THIS# ^ 4)) + (-3.79526e-12 * (#THIS# ^ 3)) + (2.13015e-7 * (#THIS# ^ 2)) + (-0.00627892803 * #THIS#) + 30.8576 |  |
| DETECTOR_11_TEMPERATURE | int(11) | Detector 11 Temperature |  |  |
| DETECTOR_12_TEMPERATURE | int(11) | Detector 12 Temperature |  |  |
| DETECTOR_13_TEMPERATURE | int(11) | Detector 13 Temperature |  |  |
| DETECTOR_14_TEMPERATURE | int(11) | Detector 14 Temperature |  |  |
| DETECTOR_15_TEMPERATURE | int(11) | Detector 15 Temperature |  |  |
| DETECTOR_16_TEMPERATURE | int(11) | Detector 16 Temperature |  |  |
| DETECTOR_1_TEMPERATURE | int(11) | Detector 1 Temperature |  |  |
| DETECTOR_2_TEMPERATURE | int(11) | Detector 2 Temperature |  |  |
| DETECTOR_3_TEMPERATURE | int(11) | Detector 3 Temperature |  |  |
| DETECTOR_4_TEMPERATURE | int(11) | Detector 4 Temperature |  |  |
| DETECTOR_5_TEMPERATURE | int(11) | Detector 5 Temperature |  |  |
| DETECTOR_6_TEMPERATURE | int(11) | Detector 6 Temperature |  |  |
| DETECTOR_7_TEMPERATURE | int(11) | Detector 7 Temperature |  |  |
| DETECTOR_8_TEMPERATURE | int(11) | Detector 8 Temperature |  |  |
| DETECTOR_9_TEMPERATURE | int(11) | Detector 9 Temperature |  |  |
| DUST_COVER_HINGE_TEMPERATURE | int(11) | (ext) temperature sensor, Dust cover hinge | 0.01171824341 * #THIS# - 256.0333 |  |
| MID_OPTICS_BRACKET_TEMPERATURE | int(11) | (ext) temperature sensor, Mid optics bracket | 0.01171824341 * #THIS# - 256.0333 |  |
| PIN_PULLER_TEMPERATURE | int(11) | (ext) temperature sensor, Pin puller | 0.01171824341 * #THIS# - 256.0333 |  |
| PRT_VOLTAGE_REFERENCE_CHANNEL_1 | int(11) | PRT voltage reference channel 1 | 0.00011444092 * #THIS# |  |
| PRT_VOLTAGE_REFERENCE_CHANNEL_2 | int(11) | PRT voltage reference channel 2 |  |  |
| PRT_VOLTAGE_REFERENCE_CHANNEL_3 | int(11) | PRT voltage reference channel 3 |  |  |
| PRT_VOLTAGE_REFERENCE_CHANNEL_4 | int(11) | PRT voltage reference channel 4 |  |  |
| PS_CURRENT_MONITOR_+12V_I | int(11) | PS Current Monitor +12V_I | 0.00008764885 * #THIS# + 0.0547 |  |
| PS_CURRENT_MONITOR_+12V_SM | int(11) | PS Current Monitor +12V_SM | 0.00008719308 * #THIS# - 0.035 |  |
| PS_CURRENT_MONITOR_+2.5V_FPGA | int(11) | PS Current Monitor +2.5V FPGA | 0.00001116497 * #THIS# - 0.0029 |  |
| PS_CURRENT_MONITOR_+2.5V_TEC | int(11) | PS Current Monitor +2.5V TEC |  |  |
| PS_CURRENT_MONITOR_+3.3V_TEC | int(11) | PS Current Monitor +3.3V TEC | 0.00021978283 * #THIS# - 0.0202 |  |
| PS_CURRENT_MONITOR_+5V | int(11) | PS Current Monitor +5V | 0.00022043371 * #THIS# + 0.0567 |  |
| PS_CURRENT_MONITOR_-12V_I | int(11) | PS Current Monitor -12V_I | 0.00008756084 * #THIS# - 0.0202 |  |
| PS_CURRENT_MONITOR_-12V_SM | int(11) | PS Current Monitor -12V_SM | 0.00008719308 * #THIS# - 0.0197 |  |
| PS_VOLTAGE_MONITOR_+12V_I | int(11) | PS Voltage Monitor +12V_I | 0.00045959473 * #THIS# |  |
| PS_VOLTAGE_MONITOR_+12V_SM | int(11) | PS Voltage Monitor +12V_SM | 0.00045959473 * #THIS# |  |
| PS_VOLTAGE_MONITOR_+2.5V_FPGA | int(11) | PS Voltage Monitor +2.5V FPGA | 0.00018310547 * #THIS# |  |
| PS_VOLTAGE_MONITOR_+2.5V_TEC | int(11) | PS Voltage Monitor +2.5V TEC | 0.00018310547 * #THIS# |  |
| PS_VOLTAGE_MONITOR_+3.3V_TEC | int(11) | PS Voltage Monitor +3.3V TEC | 0.00018310547 * #THIS# |  |
| PS_VOLTAGE_MONITOR_+5V | int(11) | PS Voltage Monitor +5V | 0.00018310547 * #THIS# |  |
| PS_VOLTAGE_MONITOR_-12V_I | int(11) | PS Voltage Monitor -12V_I | 0.00045959473 * #THIS# |  |
| PS_VOLTAGE_MONITOR_-12V_SM | int(11) | PS Voltage Monitor -12V_SM | 0.00045959473 * #THIS# |  |
| SSG_MIRROR_AMPLIFIER_PCB_TEMPERATURE | int(11) | (int) temperature sensor, SSG Mirror amplifier PCB | 0.01171824341 * #THIS# - 256.0333 |  |
| SSG_SERVO_I/O_PCB_TEMPERATURE | int(11) | (int) temperature sensor, SSG Servo I/O PCB |  |  |
| STEERING_MIRROR_BASEPLATE_TEMPERATURE | int(11) | (ext) temperature sensor, Steering mirror baseplate | 0.01171824341 * #THIS# - 256.0333 |  |
| Fore_optics_bench_3_temperature | int(11) | (ext) temperature sensor, Fore optics bracket | 0.01171824341 * #THIS# - 256.0333 |  |
| STEERING_MIRROR_TEMPERATURE | int(11) | (ext) temperature sensor, Steering mirror | 0.01171824341 * #THIS# - 256.0333 |  |
| SUN_SENSOR_HOUSING_TEMPERATURE | int(11) | (ext) temperature sensor, Sun sensor housing | 0.01171824341 * #THIS# - 256.0333 |  |
| SUN_SENSOR_PCB_TEMPERATURE | int(11) | (ext) temperature sensor, Sun sensor PCB |  |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_1 | int(11) | TEC voltage reference channel 1 | 0.00011444092 * #THIS# |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_2 | int(11) | TEC voltage reference channel 2 |  |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_3 | int(11) | TEC voltage reference channel 3 |  |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_4 | int(11) | TEC voltage reference channel 4 |  |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_5 | int(11) | TEC voltage reference channel 5 |  |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_6 | int(11) | TEC voltage reference channel 6 |  |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_7 | int(11) | TEC voltage reference channel 7 |  |  |
| TEC_VOLTAGE_REFERENCE_CHANNEL_8 | int(11) | TEC voltage reference channel 8 |  |  |
| BASE_DECK_TEMPERATURE | int(11) | (ext) temperature sensor, Base deck of SOFIE |  | 0.01171824341 * #THIS# - 256.0333 |
| CABLE_BULKHEAD_TEMPERATURE | int(11) | (ext) temperature sensor, Cable bulkhead |  | 0.01171824341 * #THIS# - 256.0333 |
| CHOPPER_HEALTH_LEFT_CHANNEL_VOLTAGE | int(11) | Voltage indicator for left side of chopper |  | 0.00004577637 * #THIS# |
| CHOPPER_HEALTH_RIGHT_CHANNEL_VOLTAGE | int(11) | Voltage indicator for right side of chopper |  |  |
| 16_CHECKSUM | int(11) | CRC16 checksum |  |  |
| CSM_BEAMSPLITTER_TEMPERATURE | int(11) |  | 0.01171824341 * #THIS# - 256.0333 |  |
| PS_CURRENT_MONITOR_+3.3V_TEC2 | int(11) | PS Current Monitor +3.3V_TEC2 | 0.00022043371 * #THIS# + 0.1241 |  |
| PS_VOLTAGE_MONITOR_+2.5V_TEC2 | int(11) | PS Voltage Monitor +2.5V_TEC2 | 0.00018310547 * #THIS# |  |
| SOFIE_RADIATOR_CENTER_TEMPERATURE | int(11) | (ext) temperature sensor, Center of radiator | 0.01171824341 * #THIS# - 256.0333 |  |
| SOFIE_RADIATOR_TOP_TEMPERATURE | int(11) | (ext) temperature sensor, Top of Radiator |  |  |
| AFT_OPTICS_BENCH_1_TEMPERATURE | int(11) | (ext) temperature sensor, Aft optics bench #1 | 0.01171824341 * #THIS# - 256.0333 |  |
| AFT_OPTICS_BENCH_2_TEMPERATURE | int(11) | (ext) temperature sensor, Aft optics bench #2 |  |  |
| AFT_OPTICS_BENCH_3_TEMPERATURE | int(11) | (ext) temperature sensor, Aft optics bench #3 |  |  |
| CSM_FAR_OPTICS_HOUSING_1_TEMPERATURE | int(11) | (ext) temperature sensor, CSM far optics housing #1 | 0.01171824341 * #THIS# - 256.0333 |  |
| CSM_FILTER_RETAINER_1_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #1 | 0.01171824341 * #THIS# - 256.0333 |  |
| CSM_FILTER_RETAINER_2_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #2 |  |  |
| CSM_FILTER_RETAINER_3_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #3 |  |  |
| CSM_FILTER_RETAINER_4_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #4 |  |  |
| CSM_FILTER_RETAINER_5_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #5 |  |  |
| CSM_FILTER_RETAINER_6_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #6 |  |  |
| CSM_FILTER_RETAINER_7_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #7 |  |  |
| CSM_FILTER_RETAINER_8_TEMPERATURE | int(11) | (ext) temperature sensor, CSM filter retainer #8 |  |  |
| DATA_ACQUISITION_PCB_1_TEMPERATURE | int(11) | (int) temperature sensor, Data acquisition PCB #1 | 0.01171824341 * #THIS# - 256.0333 |  |
| DATA_ACQUISITION_PCB_2_TEMPERATURE | int(11) | (int) temperature sensor, Data acquisition PCB #2 |  |  |
| ELECTRONICS_BOX_BASEPLATE_1_TEMPERATURE | int(11) | (int) temperature sensor, Electronics box baseplate #1 | 0.01171824341 * #THIS# - 256.0333 |  |
| ELECTRONICS_BOX_BASEPLATE_2_TEMPERATURE | int(11) | (int) temperature sensor, Electronics box baseplate #2 |  |  |
| FORE_OPTICS_BENCH_1_TEMPERATURE | int(11) | (ext) temperature sensor, Fore optics bench #1 | 0.01171824341 * #THIS# - 256.0333 |  |
| FORE_OPTICS_BENCH_2_TEMPERATURE | int(11) | (ext) temperature sensor, Fore optics bench #2 |  |  |
| SIGNAL_CONDITIONING_/_TEC_PCB_1_TEMPERATURE | int(11) | (int) temperature sensor, Signal Conditioning / TEC PCB #1 | 0.01171824341 * #THIS# - 256.0333 |  |
| SIGNAL_CONDITIONING_/_TEC_PCB_2_TEMPERATURE | int(11) | (int) temperature sensor, Signal Conditioning / TEC PCB #2 |  |  |
| SIGNAL_CONDITIONING_/_TEC_PCB_3_TEMPERATURE | int(11) | (int) temperature sensor, Signal Conditioning / TEC PCB #3 |  |  |
| SIGNAL_CONDITIONING_/_TEC_PCB_4_TEMPERATURE | int(11) | (int) temperature sensor, Signal Conditioning / TEC PCB #4 |  |  |

Pixel_Samples:

| DB Field |  | Units | Description |
| --- | --- | --- | --- |
| SAMPLE_ID | bigint(20) |  | pk |
| PACKET_ID | bigint(20) |  | Identifier to relate science data |
| SAMPLE_NUMBER | tinyint(4) |  | 1 of 7 samples. (7 samples of info per packet) |
| REPROCESSING_COUNT | int(11) |  | Indicates the number of times data has been processed |
| VERSION_ID | varchar(8) |  | Version of Level0 Software used to process data |
| PIXEL_TIME_STAMP | datetime |  | Packet time value in “2007-02-05 05:10:13” format |
| SAMPLE_MICROSECONDS | int(11) |  | Packet time value in microseconds |
| LOW_X_SUM_1 | int(11) |  | row 5 pixel sum (21 elements) in the low X ROI |
| LOW_X_SUM_2 | int(11) |  | row 6 pixel sum (21 elements) in the low X ROI |
| LOW_X_SUM_3 | int(11) |  | row 7 pixel sum (21 elements) in the low X ROI |
| LOW_X_SUM_4 | int(11) |  | row 8 pixel sum (21 elements) in the low X ROI |
| LOW_X_SUM_5 | int(11) |  | row 9 pixel sum (21 elements) in the low X ROI |
| LOW_X_SUM_6 | int(11) |  | row 10 pixel sum (21 elements) in the low X ROI |
| LOW_X_SUM_7 | int(11) |  | row 11 pixel sum (21 elements) in the low X ROI |
| HIGH_X_SUM_1 | int(11) |  | row 5 pixel sum (21 elements) in the high X ROI |
| HIGH_X_SUM_2 | int(11) |  | row 6 pixel sum (21 elements) in the high X ROI |
| HIGH_X_SUM_3 | int(11) |  | row 7 pixel sum (21 elements) in the high X ROI |
| HIGH_X_SUM_4 | int(11) |  | row 8 pixel sum (21 elements) in the high X ROI |
| HIGH_X_SUM_5 | int(11) |  | row 9 pixel sum (21 elements) in the high X ROI |
| HIGH_X_SUM_6 | int(11) |  | row 10 pixel sum (21 elements) in the high X ROI |
| HIGH_X_SUM_7 | int(11) |  | row 11 pixel sum (21 elements) in the high X ROI |
| LOW_Y_SUM_1 | int(11) |  | Column 6 pixel sum (51 elements) in the low Y ROI |
| LOW_Y_SUM_2 | int(11) |  | Column 7 pixel sum (51 elements) in the low Y ROI |
| LOW_Y_SUM_3 | int(11) |  | Column 8 pixel sum (51 elements) in the low Y ROI |
| LOW_Y_SUM_4 | int(11) |  | Column 9 pixel sum (51 elements) in the low Y ROI |
| LOW_Y_SUM_5 | int(11) |  | Column 10 pixel sum (51 elements) in the low Y ROI |
| HIGH_Y_SUM_1 | int(11) |  | Column 6 pixel sum (51 elements) in the high Y ROI |
| HIGH_Y_SUM_2 | int(11) |  | Column 7 pixel sum (51 elements) in the high Y ROI |
| HIGH_Y_SUM_3 | int(11) |  | Column 8 pixel sum (51 elements) in the high Y ROI |
| HIGH_Y_SUM_4 | int(11) |  | Column 9 pixel sum (51 elements) in the high Y ROI |
| HIGH_Y_SUM_5 | int(11) |  | Column 10 pixel sum (51 elements) in the high Y ROI |
| CENTER_SUM_1 | int(11) |  | row 1 pixel sum (31 elements) in the center ROI |
| CENTER_SUM_2 | int(11) |  | row 2 pixel sum (31 elements) in the center ROI |
| CENTER_SUM_3 | int(11) |  | row 3 pixel sum (31 elements) in the center ROI |
| CENTER_SUM_4 | int(11) |  | row 4 pixel sum (31 elements) in the center ROI |
| CENTER_SUM_5 | int(11) |  | row 5 pixel sum (31 elements) in the center ROI |
| CENTER_SUM_6 | int(11) |  | row 6 pixel sum (31 elements) in the center ROI |
| CENTER_SUM_7 | int(11) |  | row 7 pixel sum (31 elements) in the center ROI |

Science_Packets:

| DB Field | Units | Description |
| --- | --- | --- |
| PACKET_ID | bigint(20) | pk |
| PACKET_DATETIME | datetime | The time value in seconds since the Unix Epoch, extracted from the packet header |
| PACKET_MILLISECONDS | smallint(6) | Milliseconds value extracted from the packet header |
| PACKET_APID | smallint(6) | APID of packet being currently processed (0x181) |
| PACKET_CRC | int(11) | Packet CRC32 checksum |
| PACKET_IS_CLEAN | tinyint(4) | Currently, this value is always “1” |
| PACKET_SOURCE_SEQUENCE_COUNT | smallint(5) | CCSDS Packet Source Sequence Count |
| PACKET_LENGTH | smallint(5) | CCSDS Packet Length |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |
| VERSION_ID | varchar(8) | Version of Level0 Software used to process data |

SC_Fast_Sensor_Packets:

| DB Field | Units | Description |
| --- | --- | --- |
| PACKET_DATETIME | datetime | Packet time value in “2007-02-05 05:10:13” format |
| PACKET_MILLISECONDS | smallint(6) | Milliseconds value extracted from the packet header |
| PACKET_APID | smallint(6) | APID of packet being currently processed () |
| PACKET_CRC | int(11) | Packet CRC32 checksum |
| PACKET_IS_CLEAN | tinyint(4) | Currently, this value is always “1” |
| PACKET_SOURCE_SEQUENCE_COUNT | smallint(5) | CCSDS Packet Source Sequence Count |
| PACKET_LENGTH | smallint(5) | CCSDS Packet Length |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |
| VERSION_ID | varchar(8) | Version of Level0 Software used to process data |
| STAR_TRACKER_00_QUAT | float | Quaternion data from the observatory star tracker |
| STAR_TRACKER_01_QUAT | float | Quaternion data from the observatory star tracker |
| STAR_TRACKER_02_QUAT | float | Quaternion data from the observatory star tracker |
| STAR_TRACKER_03_QUAT | float | Quaternion data from the observatory star tracker |

System_Packets: look in users guide

| DB Field | Units | Description |  |
| --- | --- | --- | --- |
| PACKET_ID | bigint(20) | pk |  |
| PACKET_DATETIME | Datetime | Packet time value in “2007-02-05 05:10:13” format |  |
| PACKET_MILLISECONDS | smallint(6) | Milliseconds value extracted from the packet header |  |
| PACKET_APID | smallint(6) | APID of packet being currently processed (0x182) |  |
| PACKET_CRC | int(11) | Packet CRC32 checksum |  |
| PACKET_IS_CLEAN | tinyint(4) | Currently, this value is always “1” |  |
| PACKET_SOURCE_SEQUENCE_COUNT | smallint(5) | CCSDS Packet Source Sequence Count |  |
| PACKET_LENGTH | smallint(5) | CCSDS Packet Length |  |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |  |
| VERSION_ID | varchar(8) | Version of Level0 Software used to process data |  |
| 1553_COMMAND_HANDLER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| 1553_DATA_HANDLER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| AUTOMATION_PROCESSOR_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_BOARD_COMM_HANDLER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_COMMANDS_ACCEPTED | int(11) | C&DH Commands accepted |  |
| C&DH_COMMANDS_REJECTED | int(11) | C&DH Commands rejected |  |
| C&DH_COMMAND_EXECUTOR_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_CRITICAL_ERRORS | Int(11) | C&DH Critical error: a value of 1 indicates a X40_DUST_COVER_TIMEOUT |  |
| C&DH_DATA_ACQUISITION_HANDLER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_ERROR_HANDLER_&_FAULT_RESPONSE_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_INIT_AND_TASK_MANAGER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_QUEUE_FUNCTION_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_SELF-TEST_&_DIAGNOSTIC_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| C&DH_TASK_MANAGER_STATUS | int(11) | C&DH task manager status |  |
| CHOPPER_CONTROL_ERRORS | int(11) | There are no errors associated with this field |  |
| CODE_UPDATER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| COMMAND_PRE-PROCESSOR_ERRORS | int(11) | There are no errors associated with this field |  |
| COMMAND_RESPONSE_OPCODE | int(11) | Command response value |  |
| COMMAND_RESPONSE_VALUE | int(11) | Command response opcode |  |
| DETECTOR_CHANNEL_CONTROL_ERRORS | int(11) | There are no errors associated with this field |  |
| ENGINEERING_DATA_HANDLER_ERRORS | int(11) | There are no errors associated with this field |  |
| NUMBER_OF_LOST_MESSAGES | int(11) | Number of lost free format messages |  |
| POINTING_AND_STABILIZATION_ERRORS | int(11) | The master copy of Ptg & Stabilization errors is kept in the flight software code: ss\utils\SSQuecwp. |  |
| SCIENCE_DATA_HANDLER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SSB_STATE_&_EVENT_TABLE_MODE_WORD_3 | int(11) | SSB State & Event table word 3 (modes) |  |
| SSB_STATE_&_EVENT_TABLE_SS_ALG_STATE_WORD_4 | int(11) | SSB State & Event table word 4 (SS algorithm state) |  |
| SSB_STATE_&_EVENT_TABLE_WORD_ | int(11) | SSB State & Event table word 0 (processing rate, etc.) |  |
| SSB_STATE_&_EVENT_TABLE_WORD_1 | int(11) | SSB State & Event table word 1 (subslice #) |  |
| SSB_STATE_&_EVENT_TABLE_WORD_2 | int(11) | SSB State & Event table word 2 (20 Hz Interrupt counter) |  |
| SSB_TASK_MANAGER_STATUS | int(11) | SSB task manager status |  |
| STEERING_MIRROR_HANDLER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SUN_SENSOR_BOARD_COMM_HANDLER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SUN_SENSOR_COMMAND_EXECUTOR_ERRORS | int(11) | There are no errors associated with this field |  |
| SUN_SENSOR_CRITICAL_ERRORS | int(11) | There are no errors associated with this field |  |
| SUN_SENSOR_DATA_ACQUISITION_HANDLER_ERRORS | int(11) | There are no errors associated with this field |  |
| SUN_SENSOR_ERROR_HANDLING_&_FAULT_RESPONSE_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SUN_SENSOR_INIT_&_TASK_MANAGER_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SUN_SENSOR_QUE_FUNCTION_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SUN_SENSOR_SELF-TEST_AND_DIAGNOSTICS_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SUN_TRACKING_ALGORITHM_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| SYNC_CONTROL_REGISTER | int(11) | Sync Control Register |  |
| SYNC_RECTIFIER_PULSE_WIDTH | us | Sync Rectifier Pulse Width |  |
| SYSTEM_CRITICAL_ERRORS | int(11) | System Critical Code. The only value associated with this field is a NULL_CODE |  |
| THERMO-ELECTRIC_COOLERS_CONTROL_ERRORS | int(11) | There are no errors associated with this field |  |
| TIMED_COMMAND_PROCESSOR_ERRORS | int(11) | View the table for a complete description of the error codes and associated values |  |
| 1553_BAD_CHECKSUM_OR_ILL-FORMED_COMMAND | tinyint(1) | Error Map value 200 in the M1553_CMND_ERR error message |  |
| 1553_COMMS_FAILED | tinyint(1) | Error Map value 200 in the CDH_I_T_ERR error message |  |
| 1553_ENGINEERING_BUFFER_NOT_SERVICED | tinyint(1) | Error Map value 8 in the SCI_DATA_ERR error message |  |
| BACKUP_EVENT_COMMAND_TABLE_CHECKSUM_ERROR | tinyint(1) | Error Map value 4 in the AUTOMAT_PROC_ERR error message |  |
| BAD_IMAGE_CHECKSUM | tinyint(1) | Error Map value 2 in the CDH_I_T_ERR error message |  |
| BUFFER_NOT_AVAILABLE_FOR_DETECTOR_DATA | tinyint(1) | Error Map value 2 in the SCI_DATA_ERR error message |  |
| BUFFER_NOT_AVAILABLE_FOR_TRACKING_AND_POINTING_DATA | tinyint(1) | Error Map value 4 in the SCI_DATA_ERR error message |  |
| C&DH_BAD_IMAGE_CHECKSUM | tinyint(1) | Error Map value 2 in the CDH_I_T_ERR error message |  |
| C&DH_CODE_DUMP_REQUEST_SIZE_TRUNCATED | tinyint(1) | Error Map value 400 in the CDH_ST_DIAG_ERR error message |  |
| C&DH_COULD_NOT_SEND_HIGH_ORITY_REPLY | tinyint(1) | Error Map value 2 in the CDH_COMM_ERR error message |  |
| C&DH_COULD_NOT_SEND_SSB_DATA_MESSAGE | tinyint(1) | Error Map value 4000 in the CDH_COMM_ERR error message |  |
| C&DH_COULD_NOT_SEND_SSG_REPLY | tinyint(1) | Error Map value 10000 in the CDH_COMM_ERR error message |  |
| C&DH_COULD_NOT_SEND_STATE_&_EVENT_CHANGE_NOTICE | tinyint(1) | Error Map value 8000 in the CDH_COMM_ERR error message |  |
| C&DH_COULD_NOT_SEND_TRACKING_AND_POINTING_INFO | tinyint(1) | Error Map value 2000 in the CDH_COMM_ERR error message |  |
| C&DH_COULD_SEND_SSB_ERROR_MAP | tinyint(1) | Error Map value 20000 in the CDH_COMM_ERR error message |  |
| C&DH_DATA_DUMP_REQUEST_SIZE_TRUNCATED | tinyint(1) | Error Map value 800 in the CDH_ST_DIAG_ERR error message |  |
| C&DH_INIT_ERROR | tinyint(1) | Error Map value 800 in the CDH_COMM_ERR error message |  |
| C&DH_INVALID_COMMAND_CODE | tinyint(1) | Error Map value 4000 in the CDH_ST_DIAG_ERR error message |  |
| C&DH_INVALID_SUBPACKET_ORDER | tinyint(1) | Error Map value 100 in the CDH_COMM_ERR error message |  |
| C&DH_INVALID_TIME_SLOT | tinyint(1) | Error Map value 1000 in the CDH_COMM_ERR error message |  |
| C&DH_QUEUE_MISSING_START_SYNC | tinyint(1) | Error Map value 4 in the CDH_Queue_ERR error message |  |
| C&DH_QUEUE_OVERFLOW_ERROR | tinyint(1) | Error Map value 2 in the CDH_QUEUE_ERR error message |  |
| C&DH_QUEUE_POST_INVALID_LENGTH | tinyint(1) | Error Map value 8 in the CDH_Queue_ERR error message |  |
| C&DH_QUEUE_REQUEST_INVALID_LENGTH | tinyint(1) | Error Map value 10 in the CDH_Queue_ERR error message |  |
| C&DH_UART_FRAMING_ERROR | tinyint(1) | Error Map value 40000 in the CDH_COMM_ERR error message |  |
| C&DH_UART_PARITY_ERROR | tinyint(1) | Error Map value 80000 in the CDH_COMM_ERR error message |  |
| C&DH_UNEXPECTED_SUBPACKET_OPCODE | tinyint(1) | Error Map value 200 in the CDH_COMM_ERR error message |  |
| C&DH_UNKNOWN_RECEIVE_ERROR | tinyint(1) | Error Map value 10 in the CDH_COMM_ERR error message |  |
| CALCULATED_ATTENUATOR_SETTING_TOO_LARGE | tinyint(1) | Error Map value 1000 in the CDH_CMNDEXEC_ERR error message |  |
| CALCULATED_ATTENUATOR_SETTING_TOO_SMALL | tinyint(1) | Error Map value 2000 in the CDH_CMNDEXEC_ERR error message |  |
| CANNOT_POST_FREE_FORM_MESSAGE | tinyint(1) | Error Map value 2 in the CDH_DATA_ACQ_ERR error message |  |
| CODE_UPDATE_ERROR_CHECKSUM | tinyint(1) | Error Map value 40 in the CODEUPDATE_ERR error message |  |
| CODE_UPDATE_NEGATIVE_REQUESTED_LENGTH | tinyint(1) | Error Map value 20 in the CODEUPDATE_ERR error message |  |
| CODE_UPDATE_OD_ADDRESS_GREATER_THAN_MAXIMUM | tinyint(1) | Error Map value 8 in the CODEUPDATE_ERR error message |  |
| CODE_UPDATE_OD_ADDRESS_LESS_THAN_MINIMUM | tinyint(1) | Error Map value 8 in the CODEUPDATE_ERR error message |  |
| CODE_UPDATE_REQUESTED_LENGTH_TOO_LARGE | tinyint(1) | Error Map value 10 in the CODEUPDATE_ERR error message |  |
| COMMAND_REQUESTED_FOR_DELETION_NOT_FOUND | tinyint(1) | Error Map value 2 in the TIMED_CMND_ERR error message |  |
| COMMAND_TOO_LARGE | tinyint(1) | Error Map value 4 in the TIMED_CMND_ERR error message |  |
| COULD_NOT_SEND_COMMAND_ERROR | tinyint(1) | Error Map value 20 in the SSB_COMM_ERR error message |  |
| COULD_NOT_SEND_COMMAND_WARNING | tinyint(1) | Error Map value 10 in the SSB_COMM_ERR error message |  |
| COULD_NOT_SEND_DESIRED_AZIMUTH_AND_ELEVATION | tinyint(1) | Error Map value 2 in the STEERMIRROR_ERR error message |  |
| COULD_NOT_SEND_HIGH_ORITY_COMMAND_ERROR | tinyint(1) | Error Map value 8 in the SSB_COMM_ERR error message |  |
| COULD_NOT_SEND_HIGH_ORITY_COMMAND_WARNING | tinyint(1) | Error Map value 4 in the SSB_COMM_ERR error message |  |
| COULD_NOT_SEND_STEERING_MIRROR_COMMAND | tinyint(1) | Error Map value 4 in the STEERMIRROR_ERR error message |  |
| CRC16_CHECKSUM | int(11) | CRC16 checksum |  |
| DELETED_OLD_TIMED_COMMAND | tinyint(1) | Error Map value 8 in the TIMED_CMND_ERR error message |  |
| DIVIDE_BY_ZERO_ON_DETECTOR_COUNTS | tinyint(1) | Error Map value 200 in the CDH_CMNDEXEC_ERR error message |  |
| DUST_COVER_NOT_ARMED | tinyint(1) | Error Map value 800 in the CDH_CMNDEXEC_ERR error message |  |
| DUST_COVER_TIMEOUT | tinyint(1) | Error Map value 1 in the CDH_CRITICAL_ERR error message |  |
| EEPROM_CONTROL_BIT_NOT_SET_DURING_READ | tinyint(1) | Error Map value 8000 in the CODEUPDATE_ERR error message |  |
| EEPROM_CONTROL_BIT_NOT_SET_DURING_WRITE | tinyint(1) | Error Map value 1000 in the CODEUPDATE_ERR error message |  |
| EEPROM_DUMP_REQUEST_SIZE_TRUNCATED | tinyint(1) | Error Map value 100 in the CDH_ST_DIAG_ERR error message |  |
| EEPROM_DUMP_UNABLE_TO_COPY_DATA | tinyint(1) | Error Map value 200 in the CDH_ST_DIAG_ERR error message |  |
| EEPROM_CHKSUM_ERROR_READING_EEPROM | tinyint(1) | Error Map value 40000 in the CDH_ST_DIAG_ERR error message |  |
| EEPROM_WRITE_WOULD_CROSS_PAGE_BOUNDARIES | tinyint(1) | Error Map value 200 in the CODEUPDATE_ERR error message |  |
| EVENT_COMMAND_TABLE_EMPTY | tinyint(1) | Error Map value 1000000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_NUMBER_OUTSIDE_CURRENT_RANGE_(HIGH) | tinyint(1) | Error Map value 8000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_NUMBER_OUTSIDE_CURRENT_RANGE_(LOW) | tinyint(1) | Error Map value 4000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_SKIPPED_DUE_TO_PAST_START_TIME | tinyint(1) | Error Map value 8000000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_TABLE_NOT_PREVIOUSLY_SET_WARNING | tinyint(1) | Error Map value 2000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_TABLE_NOT_SET_UP | tinyint(1) | Error Map value 40000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_TABLE_NUMBER_OUTSIDE_RANGE_(HIGH) | tinyint(1) | Error Map value 20000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_TABLE_NUMBER_OUTSIDE_RANGE_(LOW) | tinyint(1) | Error Map value 10000 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_TABLE_NUMBER_OUTSIDE_RANGE_(STATE_1) | tinyint(1) | Error Map value 8 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_TABLE_NUMBER_OUTSIDE_RANGE_(STATE_2) | tinyint(1) | Error Map value 80 in the AUTOMAT_PROC_ERR error message |  |
| EVENT_TABLE_NUMBER_OUTSIDE_RANGE_(STATE_3) | tinyint(1) | Error Map value 800 in the AUTOMAT_PROC_ERR error message |  |
| INCORRECT_COMMAND_SIZE_ON_CODE_UPDATE | tinyint(1) | Error Map value 2 in the CODEUPDATE_ERR error message |  |
| INVALID_1553_COMMAND_PARAMETER | tinyint(1) | Error Map value 10 in the CDH_CMNDEXEC_ERR error message |  |
| INVALID_1553_MESSAGE_LENGTH | tinyint(1) | Error Map value 1 in the M1553_CMND_ERR error message |  |
| INVALID_ATTENUATOR_NUMBER | tinyint(1) | Error Map value 80 in the CDH_CMNDEXEC_ERR error message |  |
| INVALID_ATTENUATOR_PARAMETER | tinyint(1) | Error Map value 100 in the CDH_CMNDEXEC _ERR error message |  |
| INVALID_ATTENUATOR_TABLE_TYPE | tinyint(1) | Error Map value 4000 in the CDH_CMNDEXEC_ERR error message |  |
| INVALID_COMMAND_CODE | tinyint(1) | Error Map value 2 in the CDH_CMNDEXEC_ERR error message |  |
| INVALID_COMMAND_STREAM_ID_RECEIVED | tinyint(1) | Error Map value 4 in the M1553_CMND_ERR error message |  |
| INVALID_FAULT_RESPONSE_PARAMETER | tinyint(1) | Error Map value 40 in the CDH_CMNDEXEC_ERR error message |  |
| INVALID_FAULT_RESPONSE_PARAMETER_-_SSB | tinyint(1) | Error Map value 20 in the SS_CMNDEXEC_ERR error message |  |
| INVALID_FPGA_BANK | tinyint(1) | Error Map value 8000 in the CDH_ST_DIAG_ERR error message |  |
| INVALID_FPGA_BANK_-_SSB | tinyint(1) | Error Map value 8 in the SS_CMNDEXEC_ERR error message |  |
| INVALID_MEM_DUMP_OPCODE | tinyint(1) | Error Map value 2 in the CDH_I_T_ERR error message |  |
| INVALID_MODE_FOR_COMMAND | tinyint(1) | Error Map value 400 in the CDH_CMNDEXEC_ERR error message |  |
| INVALID_OPCODE_FOR_UPDATE | tinyint(1) | Error Map value 80 in the CODEUPDATE_ERR error message |  |
| INVALID_POINT_PARAMETER_-_SSB | tinyint(1) | Error Map value 40 in the SS_CMNDEXEC_ERR error message |  |
| INVALID_SCIENCE_EVENT_PROCESSOR_STATE | tinyint(1) | Error Map value 100000 in the AUTOMAT_PROC_ERR error message |  |
| INVALID_SET_EVENT_PREDICTED_COMMAND_PARAMETER | tinyint(1) | Error Map value 20 in the CDH_CMNDEXEC_ERR error message |  |
| INVALID_SSB_COMMAND_CODE | tinyint(1) | Error Map value 2 in the SS_CMNDEXEC_ERR error message |  |
| INVALID_TRACK_PARAMETER_-_SSB | tinyint(1) | Error Map value 10 in the SS_CMNDEXEC_ERR error message |  |
| LSB_EEPROM_BUSY_DURING_EEPROM_READ | tinyint(1) | Error Map value 2000 in the CODEUPDATE_ERR error message |  |
| LSB_EEPROM_BUSY_DURING_EEPROM_WRITE | tinyint(1) | Error Map value 400 in the CODEUPDATE_ERR error message |  |
| MSB_EEPROM_BUSY_DURING_EEPROM_READ | tinyint(1) | Error Map value 4000 in the CODEUPDATE_ERR error message |  |
| MSB_EEPROM_BUSY_DURING_EEPROM_WRITE | tinyint(1) | Error Map value 800 in the CDH_I_T_ERR error message |  |
| NO_CHOPPER_ENABLED | tinyint(1) | Error Map value 1 in the CDH_I_T_ERR error message |  |
| NO_MESSAGES_FROM_C&DH_-_CHANNEL_SWITCHED | tinyint(1) | Error Map value 20000 in the CDH_QUEUE_ERR error message |  |
| NO_MESSAGES_FROM_SSB_-_CHANNEL_SWITCHED | tinyint(1) | Error Map value 20000 in the SSB_COMM_ERR error message |  |
| NO_MESSAGES_FROM_SSG_-_CHANNEL_SWITCHED | tinyint(1) | Error Map value 80 in the STEERMIRROR_ERR error message |  |
| NO_TABLE_AVAILABLE_(STATE_1) | tinyint(1) | Error Map value 10 in the AUTOMAT_PROC_ERR error message |  |
| NO_TABLE_AVAILABLE_(STATE_2) | tinyint(1) | Error Map value 100 in the AUTOMAT_PROC_ERR error message |  |
| NO_TABLE_AVAILABLE_(STATE_3) | tinyint(1) | Error Map value 1000 in the AUTOMAT_PROC_ERR error message |  |
| MARY_EVENT_COMMAND_TABLE_CHECKSUM_ERROR | tinyint(1) | Error Map value 2 in the AUTOMAT_PROC_ERR error message |  |
| MARY_SSA_INIT_TABLE_EMPTY | tinyint(1) | Error Map value 20 in the CDH_I_T_ERR error message |  |
| MARY_SSA_TABLE_BAD_CHECKSUM | tinyint(1) | Error Map value 8 in the CDH_I_T_ERR error message |  |
| RESET_COMMAND_RECEIVED_IN_NON-POLLING_MODE | tinyint(1) | Error Map value 4 in the CDH_CMNDEXEC_ERR error message |  |
| SCIENCE_BUFFER_NOT_SERVICED | tinyint(1) | Error Map value 8 in the SCI_DATA_ERR error message |  |
| SECONDARY_SSA_INIT_TABLE_EMPTY | tinyint(1) | Error Map value 100 in the CDH_I_T_ERR error message |  |
| SECONDARY_SSA_TABLE_BAD_CHECKSUM | tinyint(1) | Error Map value 40 in the CDH_I_T_ERR error message |  |
| SPACECRAFT_NOT_READY_FOR_1553_ENGINEERING_DATA | tinyint(1) | Error Map value 2 in the M1553_DATA_ERR error message |  |
| SRAM_ERROR | tinyint(1) | Error Map value 4 in the CDH_I_T_ERR error message |  |
| SSB_BAD_CHECKSUM | tinyint(1) | Error Map value 2000 in the SSB_COMM_ERR error message |  |
| SSB_BAD_IMAGE_CHECKSUM | tinyint(1) | Error Map value 200 in the SS_ST_DIAG_ERR error message |  |
| SSB_CODE_DUMP_REQUEST_SIZE_GREATER_THAN_256 | tinyint(1) | Error Map value 100 in the SS_ST_DIAG_ERR error message |  |
| SSB_CODE_DUMP_TOO_FEW_PARAMETERS | tinyint(1) | Error Map value 10 in the SS_ST_DIAG_ERR error message |  |
| SSB_CODE_DUMP_TOO_MANY_PARAMETERS | tinyint(1) | Error Map value 20 in the SS_ST_DIAG_ERR error message |  |
| SSB_DATA_DUMP_REQUEST_SIZE_GREATER_THAN_256 | tinyint(1) | Error Map value 8 in the SS_ST_DIAG_ERR error message |  |
| SSB_DATA_DUMP_TOO_FEW_PARAMETERS | tinyint(1) | Error Map value 40 in the SS_ST_DIAG_ERR error message |  |
| SSB_DATA_DUMP_TOO_MANY_PARAMETERS | tinyint(1) | Error Map value 80 in the SS_ST_DIAG_ERR error message |  |
| SSB_INIT_ERROR | tinyint(1) | Error Map value 4000 in the SSB_COMM_ERR error message |  |
| SSB_INVALID_COMMAND_CODE | tinyint(1) | Error Map value 2 in the SS_ST_DIAG_ERR error message |  |
| SSB_INVALID_FPGA_BANK | tinyint(1) | Error Map value 4 in the SS_ST_DIAG_ERR error message |  |
| SSB_INVALID_SUBPACKET_ORDER | tinyint(1) | Error Map value 800 in the SSB_COMM_ERR error message |  |
| SSB_SRAM_BAD_IMAGE_CHECKSUM | tinyint(1) | Error Map value 1 in the SS_I_T_ERR error message |  |
| SSB_SRAM_ERROR | tinyint(1) | Error Map value 2 in the SS_I_T_ERR error message |  |
| SSB_UART_FRAMING_ERROR | tinyint(1) | Error Map value 8000 in the SSB_COMM_ERR error message |  |
| SSB_UART_PARITY_ERROR | tinyint(1) | Error Map value 10000 in the SSB_COMM_ERR error message |  |
| SSB_UNEXPECTED_SUBPACKET_OPCODE | tinyint(1) | Error Map value 1000 in the SSB_COMM_ERR error message |  |
| SSB_UNKNOWN_RECEIVE_ERROR | tinyint(1) | Error Map value 40 in the SSB_COMM_ERR error message |  |
| SSB_UNKNOWN_TRANSMIT_ERROR | tinyint(1) | Error Map value 80 in the SSB_COMM_ERR error message |  |
| STEERING_MIRROR_BAD_SUBPACKET_CHECKSUM | tinyint(1) | Error Map value 10 in the STEERMIRROR_ERR error message |  |
| STEERING_MIRROR_UART_FRAMING_ERROR | tinyint(1) | Error Map value 40 in the STEERMIRROR_ERR error message |  |
| STEERING_MIRROR_UART_PARITY_ERROR | tinyint(1) | Error Map value 20 in the STEERMIRROR_ERR error message |  |
| STEERING_MIRROR_UNKNOWN_RECEIVE_ERROR | tinyint(1) | Error Map value 8 in the STEERMIRROR_ERR error message |  |
| SUN_SENSOR_INIT_TABLE_CHECKSUM_ERROR | tinyint(1) | Error Map value 1 in the SUNTRACK_ERR error message |  |
| SUN_SENSOR_NOISE_FLOOR_THRESHOLD_ERROR | tinyint(1) | Error Map value 2 in the SUNTRACK_ERR error message |  |
| SUN_SENSOR_QUEUE_MISSING_START_SYNC | tinyint(1) | Error Map value 4 in the SSB_COMM_ERR error message |  |
| SUN_SENSOR_QUEUE_OVERFLOW_ERROR | tinyint(1) | Error Map value 2 in the SSB_COMM_ERR error message |  |
| SUN_SENSOR_QUEUE_POST_INVALID_LENGTH | tinyint(1) | Error Map value 10 in the SSB_COMM_ERR error message |  |
| SUN_SENSOR_QUEUE_REQUEST_INVALID_LENGTH | tinyint(1) | Error Map value 8 in the SSB_COMM_ERR error message |  |
| TABLE_ID_DOES_NOT_MATCH_WARNING | tinyint(1) | Error Map value 800000 in the AUTOMAT_PROC_ERR error message |  |
| TIMED_COMMAND_MISSING_ABSOLUTE_TIME | tinyint(1) | Error Map value 40 in the TIMED_CMND_ERR error message |  |
| TIMED_COMMAND_OLDER_THAN_CURRENT_TIME | tinyint(1) | Error Map value 10 in the TIMED_CMND_ERR error message |  |
| TIMED_COMMAND_TABLE_FULL | tinyint(1) | Error Map value 20 in the TIMED_CMND_ERR error message |  |
| TOO_FEW_PARAMETERS_ON_C&DH_CODE_DUMP | tinyint(1) | Error Map value 40 in the CDH_ST_DIAG_ERR error message |  |
| TOO_FEW_PARAMETERS_ON_C&DH_DATA_DUMP | tinyint(1) | Error Map value 10 in the CDH_ST_DIAG_ERR error message |  |
| TOO_FEW_PARAMETERS_ON_EEPROM_DUMP | tinyint(1) | Error Map value 4 in the CDH_ST_DIAG_ERR error message |  |
| TOO_MANY_PARAMETERS_ON_C&DH_CODE_DUMP | tinyint(1) | Error Map value 80 in the CDH_ST_DIAG_ERR error message |  |
| TOO_MANY_PARAMETERS_ON_C&DH_DATA_DUMP | tinyint(1) | Error Map value 20 in the CDH_ST_DIAG_ERR error message |  |
| TOO_MANY_PARAMETERS_ON_EEPROM_DUMP | tinyint(1) | Error Map value 8 in the CDH_ST_DIAG_ERR error message |  |
| UNABLE_TO_READ_EVENT_AZIMUTH_ENTRIES | tinyint(1) | Error Map value 200000 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_READ_EVENT_COMMAND_TABLE | tinyint(1) | Error Map value 80000 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_READ_EVENT_ELEVATION_ENTRIES | tinyint(1) | Error Map value 400000 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_READ_MARY_SSA_INIT_TABLE | tinyint(1) | Error Map value 10 in the CDH_I_T_ERR error message |  |
| UNABLE_TO_READ_SECONDARY_SSA_INIT_TABLE | tinyint(1) | Error Map value 80 in the CDH_I_T_ERR error message |  |
| UNABLE_TO_SEND_AZIMUTH_TABLE | tinyint(1) | Error Map value 20 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_SEND_AZIMUTH_TABLE_QUEUE_FULL | tinyint(1) | Error Map value 40 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_SEND_ELEVATION_TABLE | tinyint(1) | Error Map value 200 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_SEND_ELEVATION_TABLE_QUEUE_FULL | tinyint(1) | Error Map value 400 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_SEND_EVENT_COMMAND | tinyint(1) | Error Map value 2000000 in the AUTOMAT_PROC_ERR error message |  |
| UNABLE_TO_SEND_EVENT_COMMAND_QUEUE_FULL | tinyint(1) | Error Map value 4000000 in the AUTOMAT_PROC_ERR error message |  |
| VALID_FPGA_BANK_NOT_SELECTED | tinyint(1) | Error Map value 8000 in the CDH_ST_DIAG_ERR error message |  |
| WRITE_LENGTH_GREATER_THAN_3 | tinyint(1) | Error Map value 20000 in the CDH_ST_DIAG_ERR error message |  |
| EEPROM_ERROR_CODE_OUT_OF_RANGE | tinyint(1) | Error Map value 2 in the CDH_CMDEXEC_ERR error message |  |
| SSB_ERROR_CODE_OUT_OF_RANGE | tinyint(1) | Error Map value 2 in the SS_EH_FR_ERR error message |  |
| C&DH_BAD_IMAGE_CHECKSUM | tinyint(1) | Error Map value 1000 in the CDH_ST_DIAG_ERR error message |  |
| ATTENUATOR_SETTING_1 | int(11) | Attenuator setting #1 | #THIS# / 4095 |
| ATTENUATOR_SETTING_10 | int(11) | Attenuator setting #10 |  |
| ATTENUATOR_SETTING_11 | int(11) | Attenuator setting #11 |  |
| ATTENUATOR_SETTING_12 | int(11) | Attenuator setting #12 |  |
| ATTENUATOR_SETTING_13 | int(11) | Attenuator setting #13 |  |
| ATTENUATOR_SETTING_14 | int(11) | Attenuator setting #14 |  |
| ATTENUATOR_SETTING_15 | int(11) | Attenuator setting #15 |  |
| ATTENUATOR_SETTING_16 | int(11) | Attenuator setting #16 |  |
| ATTENUATOR_SETTING_2 | int(11) | Attenuator setting #2 |  |
| ATTENUATOR_SETTING_3 | int(11) | Attenuator setting #3 |  |
| ATTENUATOR_SETTING_4 | int(11) | Attenuator setting #4 |  |
| ATTENUATOR_SETTING_5 | int(11) | Attenuator setting #5 |  |
| ATTENUATOR_SETTING_6 | int(11) | Attenuator setting #6 |  |
| ATTENUATOR_SETTING_7 | int(11) | Attenuator setting #7 |  |
| ATTENUATOR_SETTING_8 | int(11) | Attenuator setting #8 |  |
| ATTENUATOR_SETTING_9 | int(11) | Attenuator setting #9 |  |
| SYNC_RECTIFIER_10_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #10 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_10_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #10 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_11_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #11 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_11_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #11 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_12_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #12 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_12_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #12 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_13_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #13 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_13_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #13 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_14_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #14 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_14_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #14 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_15_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #10 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_15_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #2 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_16_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #16 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_16_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #16 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_1_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #1 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_1_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #1 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_2_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #2 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_2_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #2 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_3_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #3 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_3_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #3 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_4_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #4 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_4_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #4 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_5_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #5 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_5_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #5 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_6_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #6 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_6_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #6 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_7_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #7 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_7_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #7 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_8_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #8 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_8_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #8 Rising Edge Phase Shift |  |
| SYNC_RECTIFIER_9_FALLING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #9 Falling Edge Phase Shift |  |
| SYNC_RECTIFIER_9_RISING_EDGE_PHASE_SHIFT | int(11) | Sync Rectifier #9 Rising Edge Phase Shift |  |

Template_Packet_Table:

| DB Field | Units | Description |
| --- | --- | --- |
| PACKET_ID | bigint(20) | pk |
| PACKET_DATETIME | datetime | Packet time value in “2007-02-05 05:10:13” format |
| PACKET_MILLISECONDS | smallint(6) | Milliseconds value extracted from the packet header |
| PACKET_APID | smallint(6) | APID of packet being currently processed (N/A) |
| PACKET_CRC | int(11) | Packet CRC32 checksum |
| PACKET_IS_CLEAN | tinyint(4) | Currently, this value is always “1” |
| PACKET_SOURCE_SEQUENCE_COUNT | smallint(5) | CCSDS Packet Source Sequence Count |
| PACKET_LENGTH | smallint(5) | CCSDS Packet Length |
| REPROCESSING_COUNT | int(10) | Indicates the number of times data has been processed |
| VERSION_ID | varchar(8) | Version of Level0 Software used to process data |

Tracking_Samples:

| DB Field | Units | Description |  |
| --- | --- | --- | --- |
| SAMPLE_ID |  |  | pk |
| PACKET_ID | bigint(20) |  | Identifier to relate science data |
| SAMPLE_NUMBER | tinyint(4) |  | 1 of 7 samples. (7 samples of info per packet) |
| REPROCESSING_COUNT | int(11) |  | Indicates the number of times data has been processed |
| VERSION_ID | varchar(8) |  | Version of Level0 Software used to process data |
| TRACKING_TIME_STAMP | datetime |  | Packet time value in “2007-02-05 05:10:13” format |
| SAMPLE_MICROSECONDS | int(11) |  | Packet time value in microseconds |
| LOW_X_FPA_COORDINATES | int(11) |  | Highest elevation edge of the solar image in pixels |
| HIGH_X_FPA_COORDINATES | int(11) |  | Lowest elevation edge of the solar image in pixels |
| LOW_Y_FPA_COORDINATES | int(11) |  | Left-most azimuth edge of the solar image in pixels |
| HIGH_Y_FPA_COORDINATES | int(11) |  | Right-most azimuth edge of the solar image in pixels |
| CONTROL_MIRROR_ELEVATION | int(11) |  | No longer used |
| CONTROL_MIRROR_AZIMUTH | int(11) |  | No longer used |
| TRACKING_RETURN_VALUE | int(11) |  | Return value from the on-board tracking software |
| ACTUAL_MIRROR_ELEVATION | int(11) |  | No longer used |
| ACTUAL_MIRROR_AZIMUTH | int(11) |  | No longer used |

| ### C&DH Init and Task Manager (CDH_I_T_ERR) |  |
| --- | --- |
| X41_NO_CHOPPER_ENABLED | 1 |
| X41_BAD_IMAGE_CHECKSUM | 2 |
| X41_SRAM_ERROR | 4 |
| X41_PRI_SSA_INIT_TABLE_BAD_CHECKSUM | 8 |
| X41_UNABLE_TO_READ_PRI_SSA_INIT_TABLE | 10 |
| X41_PRI_SSA_INIT_TABLE_EMPTY | 20 |
| X41_SEC_SSA_INIT_TABLE_BAD_CHECKSUM | 40 |
| X41_UNABLE_TO_READ_SEC_SSA_INIT_TABLE | 80 |
| X41_SEC_SSA_INIT_TABLE_EMPTY | 100 |
| X41_1553_COMMS_FAILED | 200 |

| 1553 Command Handler (M1553_CMND_ERR) |  |
| --- | --- |
| X42_INVALID_LENGTH | 1 |
| X42_BAD_CHECKSUM | 2 |
| X42_INVALID_SID | 4 |

| ### C&DH Self-Test & Diagnostics (CDH_ST_DIAG_ERR) |  |
| --- | --- |
| X44_MEM_DUMP_HANDLER_INVALID_OPCODE | 2 |
| X44_EEPROM_DUMP_TOO_FEW_PARAMETERS | 4 |
| X44_EEPROM_DUMP_TOO_MANY_PARAMETERS | 8 |
| X44_CDH_DATA_DUMP_TOO_FEW_PARAMETERS | 10 |
| X44_CDH_DATA_DUMP_TOO_MANY_PARAMETERS | 20 |
| X44_CDH_CODE_DUMP_TOO_FEW_PARAMETERS | 40 |
| X44_CDH_CODE_DUMP_TOO_MANY_PARAMETERS | 80 |
| X44_EEPROM_DUMP_REQUEST_SIZE_TRUNCATED | 100 |
| X44_EEPROM_DUMP_UNABLE_TO_COPY_DATA | 200 |
| X44_CDH_DATA_DUMP_REQUEST_SIZE_TRUNCATED | 400 |
| X44_CDH_CODE_DUMP_REQUEST_SIZE_TRUNCATED | 800 |
| none | 1000 |
| none | 2000 |
| X44_INVALID_CMD_CODE | 4000 |
| X44_INVALID_FPGA_BANK | 8000 |
| X44_BAD_IMAGE_CHECKSUM | 10000 |
| X44_WRITE_LEN_GT_3 | 20000 |
| X44_TEST_EE_CHKSUM_ERROR_READING_EEPROM | 40000 |

| C&DH Error Handler & Fault Response (CDH_EH_FR_ERR) |  |
| --- | --- |
| X46_INVALID_ERROR_CODE | 2 |
| X46_ERROR_CODE_OUT_OF_RANGE | 4 |

| ### Code Updater (CODEUPDATE_ERR) |  |
| --- | --- |
| CODEUPD_ERR_OK | 1 |
| X45_CODEUPD_INCORRECT_COMMAND_SIZE | 2 |
| CODEUPD_ERR_ODMIN | 4 |
| CODEUPD_ERR_ODMAX | 8 |
| CODEUPD_ERR_LENGMIN | 10 |
| CODEUPD_ERR_LENGMAX | 20 |
| CODEUPD_ERR_CKSUM | 40 |
| CODEUPD_ERR_OPCODE | 80 |
| CODEUPD_ERR_IMPLEMENT | 100 |
| CODEUPD_ERR_PAGE | 200 |
| X45_E2WRITE_ERR_LSB_EEPROM_BUSY | 400 |
| X45_E2WRITE_ERR_MSB_EEPROM_BUSY | 800 |
| X45_E2WRITE_ERR_CTRL_EEPROM_DISABLED | 1000 |
| X45_E2READ_ERR_LSB_EEPROM_BUSY | 2000 |
| X45_E2READ_ERR_MSB_EEPROM_BUSY | 4000 |
| X45_E2READ_ERR_CTRL_EEPROM_DISABLED | 8000 |

| ### C&DH Data Acquisition Handler (CDH_DATA_ACQ_ERR) |  |
| --- | --- |
| ### X4E_SYSTEM_DATA_HANDLER_GENERAL_ERROR | ### 1 |
| ### X4E_CANNOT_POST_FREE_FORM_MESSAGE | ### 2 |

| ### Sun Sensor Board Comm Handler (SSB_COMM_ERR) |  |
| --- | --- |
| X51_SS_COMM_HANDLER_GENERAL_ERROR | 1 |
| X51_DETECTED_UART_RESET | 2 |
| X51_COULD_NOT_POST_HP_COMMAND_WARN | 4 |
| X51_COULD_NOT_POST_HP_COMMAND_ERROR | 8 |
| X51_COULD_NOT_POST_COMMAND_WARN | 10 |
| X51_COULD_NOT_POST_COMMAND_ERROR | 20 |
| X51_UNKNOWN_TRANSMIT_ERROR | 40 |
| X51_UNKNOWN_RECEIVE_ERROR | 80 |
| X51_ILL_FORMED_COMMAND | 100 |
| X51_ILL_FORMED_COMMAND_NO_START | 200 |
| X51_ILL_FORMED_COMMAND_NO_END_SYNC | 400 |
| X51_INVALID_SUBPACKET_ORDER | 800 |
| X51_UNEXPECTED_SUBPACKET_OPCODE | 1000 |
| X51_BAD_CHECKSUM | 2000 |
| X51_INIT_ERROR | 4000 |
| X51_UART_PARITY_ERROR | 8000 |
| X51_UART_FRAMING_ERROR | 10000 |
| X51_NO_MSGS_FROM_SSB_RECV_CHANNEL_SWITCHED | 20000 |

| ### C&DH Queue Function (CDH_QUEUE_ERR) |  |
| --- | --- |
| X52_QUEUE_GENERAL_ERROR | 1 |
| X52_QUEUE_OVERFLOW_ERROR | 2 |
| X52_QUEUE_MISSING_START_SYNC | 4 |
| X52_QUEUE_QGET_INVALID_LENGTH | 8 |
| X52_QUEUE_QPUT_INVALID_LENGTH | 10 |
| X52_QUEUE_QLOOK_INVALID_LENGTH | 20 |

| ### C&DH Board Comm Handler (CDH_COMM_ERR) |  |
| --- | --- |
| X82_CNDH_COMM_HANDLER_GENERAL_ERROR | 1 |
| X82_COULD_NOT_POST_HP_REPLY | 2 |
| X82_COULD_NOT_POST_COMMAND_ERROR | 4 |
| X82_UNKNOWN_TRANSMIT_ERROR | 8 |
| X82_UNKNOWN_RECEIVE_ERROR | 10 |
| X82_ILL_FORMED_COMMAND | 20 |
| X82_ILL_FORMED_COMMAND_NO_START | 40 |
| X82_ILL_FORMED_COMMAND_NO_END_SYNC | 80 |
| X82_INVALID_SUBPACKET_ORDER | 100 |
| X82_UNEXPECTED_SUBPACKET_OPCODE | 200 |
| X82_BAD_CHECKSUM | 400 |
| X82_INIT_ERROR | 800 |
| X82_INVALID_TIME_SLOT | 1000 |
| X82_COULD_NOT_POST_TRACKING_AND_POINTING | 2000 |
| X82_COULD_NOT_POST_DATA_MESSAGE | 4000 |
| X82_COULD_NOT_POST_STATE_AND_EVENT | 8000 |
| X82_COULD_NOT_POST_SSG_REPLY | 10000 |
| X82_COULD_NOT_POST_SSB_ERROR_MAP | 20000 |
| X82_UART_PARITY_ERROR | 40000 |
| X82_UART_FRAMING_ERROR | 80000 |
| X82_NO_MSGS_FROM_CDH_RECV_CHANNEL_SWITCHED | 100000 |

| Sun Sensor Self-Test and Diagnostics (SS_ST_DIAG_ERR) |  |
| --- | --- |
| X84_INVALID_CMD_CODE | 2 |
| X84_INVALID_FPGA_BANK | 4 |
| X84_SSB_DATA_DUMP_REQUEST_SIZE_TRUNCATED | 8 |
| X84_SSB_DATA_DUMP_TOO_FEW_PARAMETERS | 10 |
| X84_SSB_DATA_DUMP_TOO_MANY_PARAMETERS | 20 |
| X84_SSB_CODE_DUMP_TOO_FEW_PARAMETERS | 40 |
| X84_SSB_CODE_DUMP_TOO_MANY_PARAMETERS | 80 |
| X84_SSB_CODE_DUMP_REQUEST_SIZE_TRUNCATED | 100 |
| X84_BAD_IMAGE_CHECKSUM | 200 |

### Sun Sensor Error Handlin

| ### Sun Sensor Command Executor (SS_CMNDEXEC_ERR) |  |
| --- | --- |
| X86_INVALID_CMD_CODE | 2 |
| X86_INVALID_SEQ | 4 |
| X86_INVALID_FPGA_BANK | 8 |
| X86_INVALID_TRACK_PARAM | 10 |
| X86_INVALID_FAULTRESPON_PARAM | 20 |
| X86_INVALID_POINT_PARAM | 40 |

| ### Steering Mirror Handler (STEERMIRROR_ERR) |  |
| --- | --- |
| X88_COULD_NOT_POST_DESIRED_AZ_EL | 2 |
| X88_COULD_NOT_POST_STEERING_MIRROR_CMD | 4 |
| X88_UNKNOWN_RECEIVE_ERROR | 8 |
| X88_BAD_SUBPACKET_CHECKSUM | 10 |
| X88_UART_PARITY_ERROR | 20 |
| X88_UART_FRAMING_ERROR | 40 |
| X88_NO_MSGS_FROM_SSG_RECV_CHANNEL_SWITCHED | 80 |

| Sun Sensor Error Handling & Fault Response (SS_EH_FR_ERR) |  |
| --- | --- |
| X85_ERROR_CODE_OUT_OF_RANGE | 2 |

| ### Sun Sensor Init & Task Manager (SS_I_T_ERR) |  |
| --- | --- |
| ### X81_BAD_IMAGE_CHECKSUM | ### 1 |
| ### X81_SRAM_ERROR | ### 2 |

| ### Sun Tracking Algorithm (SUNTRACK_ERR) |  |
| --- | --- |
| ### X89_SS_INIT_TABLE_CHECKSUM_ERROR | ### 1 |
| ### X89_SS_NOISE_FLOOR_THRESHOLD_ERROR | ### 2 |

| ### C&DH Command Executor (CDH_CMNDEXEC_ERR) |  |
| --- | --- |
| X48_INVALID_CMD_CODE | 2 |
| X48_INVALID_CMD_SEQ | 4 |
| X48_INVALID_FPGA_BANK | 8 |
| X48_INVALID_SET_1553_CHKSMV_PARAM | 10 |
| X48_INVALID_SET_EVENT_PRED_PARAM | 20 |
| X48_INVALID_FAULTRESPON_PARAM | 40 |
| X48_INVALID_ATTENUATOR_NUM | 80 |
| X48_INVALID_ATTENUATOR_PARAM | 100 |
| X48_DIV_BY_ZERO_DETECTOR_CNT | 200 |
| X48_INVALID_MODE | 400 |
| X48_DST_CVR_NOT_ARMED | 800 |
| X48_CAL_VAL_TO_LARGE | 1000 |
| X48_CAL_VAL_TO_SMALL | 2000 |
| X48_INVALID_ATTEN_EVENT_TYPE | 4000 |

| ### Science Data Handler (SCI_DATA_ERR) |  |
| --- | --- |
| X50_SCIENCE_DATA_HANDLER_GENERAL_ERROR | 1 |
| X50_BUFFER_NOT_AVAIL_FOR_DETECTOR_DATA | 2 |
| X50_BUFFER_NOT_AVAIL_FOR_TRACKING_AND_PTG_DATA | 4 |
| X50_1553_SCIENCE_BUFFER_NOT_SERVICED | 8 |

| ### 1553 Data Handler (M1553_DATA_ERR) |  |
| --- | --- |
| X47_ENGDAT_SC_NOT_READY | 1 |
| X47_1553_ENG_BUFFER_NOT_SERVICED | 2 |

| ### Sun Sensor Queue Function (SS_QUEUE_ERR) |  |
| --- | --- |
| X8A_QUEUE_GENERAL_ERROR | 1 |
| X8A_QUEUE_OVERFLOW_ERROR | 2 |
| X8A_QUEUE_MISSING_START_SYNC | 4 |
| X8A_QUEUE_QGET_INVALID_LENGTH | 8 |
| X8A_QUEUE_QPUT_INVALID_LENGTH | 10 |

| ### Automation Processor (AUTOMAT_PROC_ERR) |  |
| --- | --- |
| X4D_PRIMARY_EVENT_COMMAND_TABLE_CHECKSUM_ERROR | 2 |
| X4D_BACKUP_EVENT_COMMAND_TABLE_CHECKSUM_ERROR | 4 |
| X4D_STATE_1_EVENT_TABLE_NUMBER_OUTSIDE_RANGE | 8 |
| X4D_STATE_1_INTERNAL_ERROR_NO_TABLE_AVAILABLE | 10 |
| X4D_UNABLE_TO_SEND_AZ_TABLE | 20 |
| X4D_UNABLE_TO_SEND_AZ_TABLE_Q_FULL | 40 |
| X4D_STATE_2_EVENT_TABLE_NUMBER_OUTSIDE_RANGE | 80 |
| X4D_STATE_2_INTERNAL_ERROR_NO_TABLE_AVAILABLE | 100 |
| X4D_UNABLE_TO_SEND_EL_TABLE | 200 |
| X4D_UNABLE_TO_SEND_EL_TABLE_Q_FULL | 400 |
| X4D_STATE_3_EVENT_TABLE_NUMBER_OUTSIDE_RAN | 800 |
| X4D_STATE_3_INTERNAL_ERROR_NO_TABLE_AVAILABLE | 1000 |
| X4D_WARNING_EVENT_TABLE_NOT_PREVIOUSLY_SET | 2000 |
| X4D_EVENT_NUMBER_OUTSIDE_CURRENT_RANGE_LOW | 4000 |
| X4D_EVENT_NUMBER_OUTSIDE_CURRENT_RANGE_HIGH | 8000 |
| X4D_EVENT_TABLE_NUMBER_OUTSIDE_RANGE_LOW | 10000 |
| X4D_EVENT_TABLE_NUMBER_OUTSIDE_RANGE_HIGH | 20000 |
| X4D_ERROR_EVENT_TABLE_NOT_SET_UP | 40000 |
| X4D_UNABLE_TO_READ_EVT_COMMAND_TABLE | 80000 |
| X4D_INTERNAL_ERROR_INVALID_SEP_STATE | 100000 |
| X4D_UNABLE_TO_READ_EVT_AZ_ENTRIES | 200000 |
| X4D_UNABLE_TO_READ_EVT_EL_ENTRIES | 400000 |
| X4D_WARNING_TABLE_ID_DOESNT_MATCH | 800000 |
| X4D_SCI_EVT_CMD_TABLE_EMPTY | 1000000 |
| X4D_UNABLE_TO_SEND_EVENT_COMMAND | 2000000 |
| X4D_UNABLE_TO_SEND_EVENT_COMMAND_Q_FULL | 4000000 |
| X4D_EVENT_TIME_IS_PAST_EVENT_SKIPPED | 8000000 |

| ### Timed Command Processor (TIMED_CMND_ERR) |  |
| --- | --- |
| X4C_COMMAND_REQUESTED_FOR_DELETION_NOT_FOUND | 1 |
| X4C_COMMAND_TOO_LARGE | 2 |
| X4C_ERROR_DELETED_OLD_TIMED_COMMAND | 4 |
| X4C_WARNING_TIMED_COMMAND_OLDER_THAN_CURRENT_TIME | 8 |
| X4C_TIMED_COMMAND_TABLE_FULL | 10 |
| X4C_TIMED_COMMAND_MISSING_ABSOLUTE_TIME | 20 |
| X4C_TIMED_COMMAND_MISSING_ABSOLUTE_TIME | 40 |