| Small Explorer Program Aeronomy of Ice in the Mesosphere (AIM) |  |
| --- | --- |
| AIM Science Data System Interface Control Document |  |
| GATS Document No: AIM-SDS-ICD-03-1 | Date: April 1, 2005 |

| Revisions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Rev | Description of Change | By | Approved | Date |
| 0 | Version 0 for PDR | J. Burton |  | 01/21/04 |
| 1 | Version 1 for CDR | J. Burton |  | 10/12/04 |
|  | Version 1 for MOR | J. Burton |  | 04/01/05 |

Signature Page

Prepared by:

___________________________

John C. Burton

AIM Science Data Systems Manager

Concurred by:

___________________________	___________________________

David Welch	Cindy Russell

AIM Mission Operations Manager	CIPS/CDE Data Systems Manager

___________________________

Randal Davis

AIM Ground Data Systems Manager

Approved by:

___________________________	___________________________

Mary Bolton	Scott M. Bailey

AIM Project Manager	AIM Deputy Principal Investigator

___________________________

James M. Russell III

AIM Principal Investigator

Table of Contents

1	Introduction	1

1.1	Identification	1

1.2	Scope	1

2	Related Documents	1

3	AIM Data System Overview	1

3.1	Data System Concept	2

3.2	Interfaces	2

3.2.1	Meta-data Messages	3

3.2.2	Transfer Protocols	4

3.2.3	File Format	4

3.2.4	Naming Convention	4

3.2.5	Global File Attributes	4

3.2.6	NetCDF Variable Attributes	4

4	SDS Interface Descriptions	5

4.1	Interface SDS-II-1	5

4.1.1	Data Processing Center URL (DPC_URL)	6

4.1.2	Product Availability Notice (PAN)	6

4.1.3	Science Data Product URL (SDP)	6

4.1.4	Data Product Specification (DPS)	6

4.1.5	Problem Report	7

4.1.6	Change Request	7

4.1.7	Change Notification	7

4.2	Interface SDS-II-2	7

4.2.1	Product Availability Notice Response (PAN_RES)	7

4.2.2	Common Volume Observation Request (CVO_RQS)	7

4.3	Interface SDS-II-3	9

4.3.1	Data Transfer Notice	9

4.4	Interface SDS-II-4	10

4.5	Interface SDS-II-5	8

4.5.1	Data Product Specification	8

4.5.2	Product Availability Notice	8

4.6	Interface SDS-EI-1	8

4.7	Interface SDS-EI-2	9

Appendix A	File Conventions	11

A.1	Naming Conventions	11

A.2	Global Attributes / Standard Header	11

A.3	NetCDF File Variable Attributes	11

Appendix B	Message Formats / Contents	14

B.1	Data Processing Center URL Message Format	14

B.2	Product Availability Notice Message Format	14

B.3	Product Availability Notice Response Message Format	15

B.4	Common Volume Observation Request Format	15

B.5	Data Transfer Notice Message Format	16

B.6	Document Upload Message	17

B.7	Change Management Messages	17

Appendix C	Data Product Specification	20

C.1	Data Product Specification	20

C.2	Description of Field Value choices	21

Appendix A	Acronyms and Definitions	24

# Introduction

## Identification

This Interface Control Document (ICD) is a required deliverable under the Aeronomy of Ice in the Mesosphere (AIM) Contract, NAS 5-03074.

## Scope

This Interface Control Document (ICD) describes the interfaces between the AIM Project Data Center (PDC) and the CIPS/CDE/SOFIE instrument Data Processing Centers (DPCs) that are required to support instrument data processing, analysis of the individual instrument’s telemetry, and dissemination and fusion of the science data products. This document defines the physical and operational interfaces, communications protocols, and message formats for data and information exchanged between the AIM PDC and the SOFIE, CIPS, and CDE DPCs. This includes the support data, product status and science data products.

# Related Documents

- AIM Data Management Plan, G&A Technical Software, Inc., AIM-DMP-03-1.V0, Dec 2003

- AIM MOC External Interface Control Document, CU/LASP,???

# AIM Data System Overview

Figure 3-1 AIM Data System

## Data System Concept

The AIM Data System consists of two primary elements, the Mission Operations Data System (MODS) and the Science Data System (SDS) as shown in Figure 3-1 AIM Data System. The MODS elements, consisting of the Ground Station, Mission Operations Center (MOC), and instrument Payload Operation Centers (POCs) are responsible for the command, control and monitoring of the AIM spacecraft and instruments. The SDS elements, which include the Project Data Center (PDC), the instrument Data Processing Centers (DPCs) and the portion of the POCs engaged in telemetry storage and distribution, are responsible for the transformation of raw instrument data into the science data products described in the AIM Data Management Plan.  This document defines the routine interfaces between the PDC and the instrument DPCs in terms of processes, data flow, stream service, file definitions, and file naming conventions.  The interfaces between the MODS elements are defined in the AIM MOC External Interface Control Document.

## Interfaces

The internal SDS interfaces are defined based on using the public Internet as a transfer medium. The Internet is considered insecure, requiring measures to be taken to protect resources and ensure the authenticity and validity of communications. Each SDS facility is responsible for implementing security measures in accordance to NASA ADP security guidelines. The interfaces defined in this document take these security precautions into consideration.

The transfer of information among the various SDS facilities is based on the generalized concept of separating information into data files and meta-data messages. Meta-data messages will be transferred as XML formatted email or directly input as fields in a web form. Some meta-data messages will reference data files via a URL or direct attachment to the meta-data message, other meta-data messages will be complete in and of themselves. Below is a list of the major categories of meta-data messages.

- Data Product messages – These messages typically have a data file associated with them either via URL or attachment. The data files are SDS controlled and are described by a Data Product Specification. Data Product messages are considered routine and will be exchanged via email.

- Supplemental Data messages – These messages have a data file attached to them. The data files are not SDS controlled and do not have an associated Data Product Specification. Supplemental Data messages are considered on-demand and for the most part will be handled via electronic web forms. An email interface will also be provided to help automate the uploading of PMC images captured by EPO sites.

- Configuration Management messages – These messages contain important information concerning changes in the configuration of the SDS. Configuration Management messages are considered on-demand and for the most part will be handled via electronic web forms. Due to the amount of information involved, an optional email interface will also be provided for transferring the Data Product Specification and Data Processing Center URL.

- Document messages – These messages contain information about AIM mission documentation or publications derived from the AIM mission. They are considered on-demand and handled via electronic web forms.

The interfaces described in this document are defined by four characteristics: message (and format), transfer protocols, content, and associated data files. For instances where filenames are needed a standard set of naming conventions is given. The characteristics are unique to each interface and described in each of the following sections.

### Meta-data Messages

Table 3-1 provides a description of the meta-data messages that are transferred between SDS facilities. The formats of the messages are included in this document. The format of the associated data files under SDS control will be specified using a Data Product Specification and stored in the PDC data catalog.

Table 3-1 Meta-data Message Description

| Message | Message Type | Format | Transfer Class | Associated data files |
| --- | --- | --- | --- | --- |
| Product Availability Notice (PAN) | Data Product | Table B-6 | Routine | Mission Status Report Data Quality Report Science Data Products |
| PAN Response (PAN_RES) | Data Product | Table B-7 | Routine |  |
| Common Volume Observation Request (CVO_RQS) | Data Product | Table B-8 | Routine |  |
| Data Transfer | Supplemental Data | Table B-9 | Routine / On-demand | PMC Images Correlative Data Validation Data |
| Problem Report | Configuration Management | Table B-11 | On-demand | N/A |
| Change Request | Configuration Management | Table B-12 | On-demand | N/A |
| Change Notification | Configuration Management | Table B-13 | On-demand | N/A |
| Data Product Specification | Configuration Management | Table C-14 | On-demand | ASCII files – description of field contents and formats NetCDF – CDL file for this product Other – format/content definition |
| Data Processing Center URL | Configuration Management | Table B-5 | On-demand | N/A |
| Document Upload | Document | Table B-10 | On-demand | Mission Document Publication Abstract |

### Transfer Protocols

The transfer of meta-data messages among the various SDS elements will occur via either digitally signed XML formatted email (SMTP), or via web based forms. Instrument science data products used by the PDC to generate CVO routine products will reside at the producing DPC and be downloaded by the PDC via HTTP or SCP. External users (general public and science community) will access science data via HTTP or FTP using URL links provided by the PDC, or directly from the instrument DPCs.

### File Format

The formats for files transferred between SDS facilities are summarized below.

- Level 1 and higher instrument data products – NetCDF

- Correlative, collaborative, or validation data products – NetCDF, or well formatted ASCII for small quantities of data.

- Supplemental PMC images will be in PNG or JPEG format.

- Status reports and other small products that are best represented in text should be transmitted in well formatted ASCII files as negotiated

- Survey products containing images – GIF, PNG, JPEG, Postscript, PDF

- Survey products containing animation – MPEG

### Naming Convention

In order to specify the file and message naming convention it is helpful to specify the standard conventions that this document uses to describe a name. These standards are specified in Table A-2 Naming Conventions.

### Global File Attributes

Global attributes or standard headers are required for each SDS controlled NetCDF and ASCII file, respectively. In ASCII files these attributes should take the XML compatible form of <keyword> value </keyword>. The proposed set of attributes or keyword/value pair can be found in Table A-3 Global Attributes / Standard Header.

### NetCDF Variable Attributes

In addition to the global attributes described above, each variable in a NetCDF file should be documented with a standard set of attributes. These variable attributes will be used by the NSSDC to ingest the AIM mission final archive, and by others interested in using the data. A list of recommended variable attributes and descriptions can be found in Table A-4 NetCDF File Variable Attributes.

# SDS Interface Descriptions

The AIM PDC is the central focus point for communications within the SDS. The communications interfaces shown in Figure 4-2 SDS Data Interfaces are the subject of this document.  The PDC – DPC interfaces are the same for the CDE, CIPS, and SOFIE DPCs, so only a generic DPC is shown in Figure 4-2.

Figure 4-2 SDS Data Interfaces

## Interface SDS-II-1

SDS-II-1 is the SDS internal interface from the instrument DPCs to the PDC used to transfer the following information:

| Message | Data File | Data Protocol | Message Protocol |
| --- | --- | --- | --- |
| Data Processing Center URL (DPC_URL) | N/A | N/A | Email / web form |
| Product Availability Notice (PAN) | Science Data Product (SDP) | URL | Email / web form |
| Product Availability Notice | Data Quality Report | Attach | Email / web form |
| Data Product Specification (DPS) | CDL file / format description | Attach | Email / web form |
| Problem Report | N/A | N/A | web form |
| Change Request | N/A | N/A | web form |
| Change Notification | N/A | N/A | web form |

### Data Processing Center URL (DPC_URL)

The Data Processing Center URL can be transferred to the PDC by the instrument DPC via XML formatted email or web form. It will be used by the PDC to provide links to the science data production facilities’ web sites. The web site should contain product descriptions, instrument information, and such as outlined in AIM Data Management Plan. The message is defined in Table B-5 DPC URL Message

### Product Availability Notice (PAN)

The Product Availability Notice (PAN) will be sent to the PDC by the instrument DPC to provide the metadata for a data product(s). This information will be stored in the Mission Data Catalog and will be used when the AIM project and the general public request data products. The producers of the data products will send a PAN to the PDC to indicate the generation of a new data product, the update to the metadata of an existing data product, or the deletion of an existing data product. A message definition is provided in Table B-6. Because each data product produced for the AIM project requires either global attributes (NetCDF files) or a standard header (ASCII Files), the majority of information contained in the Product Availability Notice can be pulled directly from these attributes or standard headers. A single PAN can be used to send metadata for multiple data products. A PAN will also be submitted for the science data quality report., which is a text report concerning the quality of the science data processed.

### Science Data Product URL (SDP)

Based on the URL specified in the PAN, the PDC will open a secure connection, (SCP/SFTP) to the instrument DPC and copy the SDP to the local file system. The SDP will be a NetCDF file containing the global attributes described in Table A-3 and data as agreed upon by the instrument Data Manager and the AIM Data Manager. For higher level data products, JPEG, PNG, Postscript or other format listed in the AIM Data Management Plan is acceptable.

### Data Product Specification (DPS)

Each data product controlled by the SDS will have a data product specification stored in the PDC data catalog. These specifications also contain a description of the format and content of the data product as described in Table C-14 Data Product Specification. In the case of a netCDF file the CDL file used to define the netCDF file is used. These specifications will be used as aids for data location, data product descriptions and as an aid to interface control.

### Problem Report

The problem report is an integral part of the corrective action process. When a problem is found with data products, documents, or other items under SDS control, the first step is to notify the SDS manager via a problem report. A problem report is submitted electronically to the PDC via an electronic form on a PDC web page.  Actions subsequent to the receipt of the problem report are documented in the AIM Data Management Plan. The contents of the problem report form are described in Table B-11.

### Change Request

Like the problem report, the change request is part of the corrective action process. It is also submitted electronically to the SDS manager, via electronic web form. Subsequent actions are also documented in the AIM Data Management Plan. The contents of the change request form are listed in Table B-12.

### Change Notification

Before any change is made to configuration items, SDS facilities and users will be notified via a change notification. Facilities are also encouraged to use this process for minor releases of data product versions as well. At least one week before a change goes into effect, the facility responsible for making the change should submit a change notification to the SDS manager. This form will be available electronically. The contents of this form are described in Table B-13.

## Interface SDS-II-2

SDS-II-1 is the SDS internal interface from the PDC to the instrument DPCs used to transfer the following information:

| Message | Data File | Data Protocol | Message Protocol |
| --- | --- | --- | --- |
| Product Availability Notice Response (PAN_RES) | N/A | N/A | Email |
| Common Volume Observation Request (CVO_RQS) | N/A | N/A | Email |

### Product Availability Notice Response (PAN_RES)

Upon receipt of a Product Availability Notice (PAN) from the DPC, the PDC will check the authenticity of the digital signature of the enclosing e-mail. If the signature is valid the PDC will the decrypt the e-mail contents as needed and test the validity of each URL contained in the PAN. The results of this authentication and validation procedure are sent back to the producer via e-mail with a copy of the message stored on the PDC site. The format of this response file is given in Table B-7.

### Common Volume Observation Request (CVO_RQS)

After the SOFIE DPC have processed its data for a given time period and the appropriate PAN file has been successfully submitted to the PDC, the PDC will download the SOFIE data product file to a temporary storage area and extract the Line of Sight (LOS) information for each observation. Also extracted will be meta-data information including date/time and location. The PDC will then send this information to the CIPS DPC in the form of a CVO request. The format of the CVO request is given in Table B-8 Common Volume Observation Request. The CIPS DPC will respond by generating a data product netCDF file containing data coincident with the SOFIE LOS information. Once the data product file has been produced, the CIPS DPC will respond with a PAN referencing the CVO request and informing the PDC of the location of the data product.

.

## Interface SDS-II-3

SDS-II-3 is the SDS internal interface from the MOC to the PDC used to transfer the following information:

| Message | Data File | Data Protocol | Message Protocol |
| --- | --- | --- | --- |
| Data Product Specification (DPS) | CDL file / format description | Attach | Email / web form |
| Product Availability Notice | Mission Status Report | Attach | Email / web form |
| Product Availability Notice | Data Quality Report | Attach | Email / web form |
| Problem Report | N/A | N/A | web form |
| Change Request | N/A | N/A | web form |
| Change Notification | N/A | N/A | web form |

### Data Product Specification

The MOC will provide a DPS for the Mission Status Report and the Data Quality Report, including a description of the format and content of the two reports.

### Product Availability Notice

The MOC will use a PAN with the Mission Status Report or the Data Quality Report attached as the transfer mechanism for the reports.

## Interface SDS-EI-1

SDS-EI-1 is the SDS interface between the PDC and the NSSDC for the final archive of the AIM mission data and documents. The data will be contained in standard Tape Archive (TAR) files. The contents of the final archive transferred to the NSSDC will include:

- Tar file of the XML formatted dump of the PDC data catalog.

- Tar file of PDC documentation files and directory structure.

- Tar file of CVO data files and directory structure.

- Tar file of SOFIE data files and directory structure (from SOFIE DPC).

- Tar file of SOFIE documentation files and directory structure.

- Tar file of CIPS data files and directory structure (from CIPS DPC).

- Tar file of CIPS documentation files and directory structure.

- Tar file of CIPS data files and directory structure (from CIPS DPC).

- Tar file of CIPS documentation files and directory structure.

## Interface SDS-EI-2

SDS-EI-2 is the SDS interface between the general public and science community and the PDC. It is similar to SDS-II-4 in that no meta-data messages will be transferred from the PDC to general public or science community, but rather the PDC will serve AIM mission documents and data via HTML web pages and URL links. Textual and graphical information will be viewed directly by a standard web browser, and document and data product files can be downloaded via HTTP or FTP protocols.

## Interface SDS-EI-3

SDS-II-3 is the SDS internal interface from EPO, Science Team, and Co-investigator affiliated systems to the PDC used to transfer the following information:

| Message | Data File | Data Protocol | Message Protocol |
| --- | --- | --- | --- |
| Data Transfer Notice | Supplementary Data | Attach | Email / web form |
| Problem Report | N/A | N/A | web form |
| Change Request | N/A | N/A | web form |
| Change Notification | N/A | N/A | web form |
| Document Upload | Document | Attach | web form |

Documents, publication references and abstracts will be uploaded to the PDC via a web form.

### Data Transfer Notice

Users affiliated with the AIM project can upload supplementary data for storage by the PDC. This data should be complementary to and coincident with AIM data. Supplementary data would include images of PMC captured by some of the EPO webcams and data used to validate AIM data products. The Data Transfer Notice is described in Table B-9. Normally the transfer of supplementary data to the PDC will be handled via a web form upload. The Data Transfer Notice can be used to automate the process as needed. The web form would request the same information given in Table B-9.

### Document Upload

As stated in the AIM Data Management Plan, users of the AIM system require access to relevant descriptions of research results for the AIM mission. The document upload mechanism allows individual users to upload documents concerning various aspects of the AIM mission, as well as abstracts and references to articles published in journals.

## Interface SDS-EI-4

SDS-II-3 is the SDS internal interface used to transfer information from the PDC to the EPO, Science Team, and Co-investigator affiliated systems. No meta-data messages will be transferred from the PDC to the EPO, Science Team, and Co-investigator systems, but rather the PDC will serve AIM mission documents and data via HTML web pages and URL links. Textual and graphical information will be viewed directly by a standard web browser, and document and data product files can be downloaded via HTTP or FTP protocols

# File Conventions

## Naming Conventions

Table A-2 Naming Conventions

| Descriptor | Definition | Example(s) |
| --- | --- | --- |
| inst | Instrument identification string | CDE,CIPS,SOFIE, |
| APID | 3 character hexadecimal Application Process Identifier |  |
| ####, ###, ##, # | Padded 4 digit revision ID, padded 3 digit version ID, etc. | 0001, 001 |
| yyyy | UT Year | 2003 |
| doy | UT Day of year | 030 |
| hh | UT hour of day | 12 |
| mm | UT minutes of hour | 06 |
| ss | UT seconds of minute | 59 |
| starttime | UT yyyydoyhhmmss | 2003211000000 |
| stoptime | UT yyyydoyhhmmss | 2003212000000 |

## Global Attributes / Standard Header

Table A-3 Global Attributes / Standard Header

| NetCDF Attribute or XML Keyword | Req/Opt | Example |  | Meaning |
| --- | --- | --- | --- | --- |
|  |  | NetCDF | XML (ASCII) |  |
| Title | R | Zonal Mean Mixing Ratios | <title> Zonal Mean Mixing Ratios </title> | A character array providing a succinct description of what is in the data set. |
| DP_Type | R | Level 2 | <dp_type> Level 2 </dp_type> | This is the type that appears in the data product specification that describes this file |
| Source | R | SOFIE DPC | <source> SOFIE DPC </source> | The person or facility that created this product |
| Mission | R | AIM | <Mission> AIM </Mission> | Always AIM |
| DP_Version | R | 01.01 | <dp_version> 01.01 </dp_version> | Indicates how many times the content or format for date product type has changed |
| PF_Version | R | 01.01 | <pf_version> 01.01 </pf_ersion> | Indicates how many times the format of the data type has changed |
| SW_Version | R | 01.01 | <sw_version> 01.01 </sw_version> | Version number of the software that generated this product |
| SW_Name | R | SOFIE_L2 | <sw_name> SOFIE_L2 </sw_name> | Name of the software that created this product |
| Calib_Version | R | 01.01 | <calib_version> 01.01 </calib_version> | Version of the calibration used to process these data |
| Description | O | This is a description | <description> This is a description </description> |  |
| History | O |  | <history> <entry> <timestamp></timestamp> <username></username> <sw_name></sw_name> <cmd_args> </cmd_args> </entry> </history> | Taken from NetCDF Manual. A global attribute for an audit trail. This is a character array with a line for each invocation of a program that has modified the data set. Well-behaved generic NetCDF applications should append a line containing: date, time of day, username, program name, and command arguments. |
| Name | R | SOFIE_yyyydoy_##.nc | <filename> SOFIE__yyyydoy_##.nc </filename> | The name of the file or message containing these global attributes / header |
| Gen_Date | R | yyyydoyhhmmss | <gen_date> yyyydoyhhmmss </gen_date> | Date that this data product was created |
| Standard Header Delimiter | R for XML |  | <standard_header> </standard_header> | For an ASCII file this string is required to appear on a single line after the last keyword/value pair of the standard header and before the beginning of the data in the file. |

## NetCDF File Variable Attributes

Table A-4 NetCDF File Variable Attributes

| Attribute Name | Req/Opt | Description (from NetCDF User’s Guide) |
| --- | --- | --- |
| description | R | A character string providing a succinct description of the variable and any quantity the variable depends on. This information needs to be complete enough that users can select variables of interest based only on this value. |
| units | R | A character string that specifies the units used for the variable’s data |
| long_name | R | A long descriptive name to be used for labeling plots. |
| valid_min | R | A scalar value specifying the minimum valid value for this variable |
| valid_max | R | A scalar value specifying the maximum valid value for this variable |
| scale_factor | O | If present for a variable, the data are to be multiplied by this factor after the data are read by the application that accesses the data. |
| add_offset | O | If present for a variable, this number is to be added to the data after it is read by the application that accesses the data. If both scale_factor and add_offset attributes are present, the data are first scaled before the offset is added. |
| missing_value | O | This attribute can be a scalar or vector containing values indicating missing data. These values should be outside the range defined by (valid_min, valid_max) so that generic applications will trest them as missing. |

# Message Formats / Contents

## Data Processing Center URL Message Format

Table B-5 DPC URL Message

| Naming Convention | inst_##.pur |  |
| --- | --- | --- |
| Transfer Protocol | SMTP from DPC to PDC. |  |
| Availability | Launch |  |
| Format | ASCII file, XML format |  |
| Contents |  |  |
| Standard Header | For contents of standard header see Table A-3 |  |
| Field Name | Description | Delimiter |
| E-mail Address | E-mail address to be used in sending the PANs to the PDC | <email_address> </email_address> |
| Complete URL | DPC Product Page address (example: HTTP://sofie.gats-inc.com/data_products.htm) | <url> </url> |

## Product Availability Notice Message Format

Table B-6 Product Availability Notice

| Naming Convention | inst_yyyydoyhhmmss_##.pan where inst is the data product producer (e.g., CIPS, CDE, SOFIE) yyyydoyhhmmss is the PAN generation datetime stamp, ## is the revision number of the PAN definition The revision number will be 01 initially and will be increased if the PAN definition changes. |  |  |  |
| --- | --- | --- | --- | --- |
| Transfer Protocol | SMTP from DPC to PDC |  |  |  |
| Availability | This message should be transferred as soon as the data product has been created or revised. |  |  |  |
| Format | ASCII file, XML format |  |  |  |
| Contents |  |  |  |  |
| Standard Header | For contents of standard header see: Table A-3 |  |  |  |
|  | For each data product for which status is being reported: |  |  |  |
| FieldName | Required or Optional | Comments | Valid Values | Delimiter |
| Data Product Filename | R | The filename of the data product (unique id of a specific data product) |  | <filename> </filename> |
| PAN Action Flag (PAF) | R | A flag used to indicate the purpose of the PAN (i.e., New, Update, or Delete) | N = New Data Product U = Update Metadata D = Delete Data Product | <pan_action> </pan_action> |
| Data Product Type Name | R if PAF = U or D | The type of data product (e.g., SOFIE Level 2 Data) | See DMP Table 9-1 | <dp_ type> </dp_ type> |
| Version Number | R if PAF = N | The version number of the data product |  | <dp_revision> </dp_revision> |
| Complete URL | R | The product file WWW location |  | <dp_url> </dp_url> |
| Data Start Time | R if PAF = N | Start time of the data collection |  | <start_time> </start time> |
| Data End Time | R if PAF = N | End time of the data collection |  | <end_time> </end_time> |
| Earth location of observation (GBO) | O | For mobile GBO to report location of instrument | Lat, Lon ,Alt | <gbo_location> </gbo_location> |
| Comments | O | Additional Comments |  | <comments> </comments> |

## Product Availability Notice Response Message Format

Table B-7 Product Availability Notice Response

| Naming Convention | inst_yyyydoyhhmmss_##.pan.res The name is simply the fname of the PAN with the .res extension attached |  |  |  |
| --- | --- | --- | --- | --- |
| Transfer Protocol | SMTP from PDC to DPC |  |  |  |
| Availability | This file should be transmitted as soon as the PAN has been processed. |  |  |  |
| Format | ASCII file, XML format |  |  |  |
| Contents |  |  |  |  |
| Standard Header | For contents of standard header see: Table A-3 |  |  |  |
| PAN Authenticity Results | Results from the digital signature authenticity check and decryption of contents as needed. |  |  |  |
|  | For each data product for which status is being reported: |  |  |  |
| Column Name | Required or Optional | Comments | Valid Values | Delimiter* |
| Data Product Filename | R | The filename of the data product (unique id of a specific data product) |  | <filename> </filename> |
| Complete URL | R | The product file WWW location |  | <dp_url> </dp_url> |
| URL Validity Test Results | R | Either OK or the error code returned by the URL server |  | <url_valid> </url_valid> |
| URL Validity Test Messages | O | Error message returned by the URL server if there was a problem accessing the file specified in the URL |  | <url_message> </url_message> |

## Common Volume Observation Request Format

Table B-8 Common Volume Observation Request

| Naming Convention | PDC_yyyydoyhhmmss_##.cvo where yyyydoyhhmmss is the CVO generation date/time stamp, ## is the revision number of the CVO definition The revision number will be 01 initially and will be increased if the CVO definition changes. |  |  |  |
| --- | --- | --- | --- | --- |
| Transfer Protocol | SMTP from PDC to DPC |  |  |  |
| Availability | This file should be available as soon as the data product has been created or revised. |  |  |  |
| Format | ASCII file, XML format |  |  |  |
| Contents |  |  |  |  |
| Standard Header | For contents of standard header see: Table A-3 |  |  |  |
| LOS Vector | A vector of comma separated values defining the SOFIE line of sight for each observation. Delimited with <sofie_los></sofie_los> |  |  |  |
|  | The remainder of the CVO request file will consist of two entries defining the starting and ending points of the data box containing the coincident measurements (CVO). The starting point entry is delimited by <data_start></data_start> and the ending point is delimited by <data_end></data_end>. Each entry will consist of: |  |  |  |
| Field Name | Required or Optional | Comments | Valid Values | Delimiter* |
| Date/time stamp | R | Standard UTC date/time string of the data box corner in yyyydoyhhmmss format |  | <time_stamp> </time_stamp> |
| Latitude | R | Latitude of the data box corner |  | <latitude> </latitude> |
| Longitude | R | Longitude of the data box corner |  | <longitude> </longitude> |
| Altitude | O | Altitude of the data box corner |  | <altitude> </altitude> |

## Data Transfer Notice Message Format

Table B-9 Data Transfer Notice Message

| Transfer Protocol | SMTP from GBO to PDC |  |  |  |
| --- | --- | --- | --- | --- |
| Availability | This file should be available as soon as the data product has been created or revised. |  |  |  |
| Format | ASCII file, XML format |  |  |  |
| Contents |  |  |  |  |
| Standard Header | None |  |  |  |
|  | One data file per data transfer notice |  |  |  |
| FieldName | Required or Optional | Comments | Valid Values | Delimiter |
| Upload Source Name | R | The name of the individual or institution uploading the data file |  | <u_source> </u_source> |
| Data Source Name | R | The name of the source of the data |  | <dp_source> </dp_source> |
| Data File Name | R | The name of the data file being uploaded with the data transfer notice |  | <filename> </filename> |
| Data Content Classifier | R | The classification of the data in the file. | See Table C-16 | <content_class> </content_class> |
| File Contents | R | A description of the format / contents of the data file. |  | <contents> </contents> |
| Earth location of observer | R | For ground based observer to report location of instrument | Lat, Lon ,Alt | <gbo_location> </gbo_location> |
| Direction of observation | R | For ground based observer to report 3-D look direction of instrument |  | <look_direction> </look_direction> |
| Comments | O | Additional Comments |  | <comments> </comments> |
| The remainder of the data transfer notice will consist of two entries defining the starting and ending points of the data box containing the data. The starting point entry is delimited by <data_start></data_start> and the ending point is delimited by <data_end></data_end>. Each entry will consist of: |  |  |  |  |
| Date/time stamp | O | Standard UTC date/time string of the data box corner in yyyydoyhhmmss format |  | <time_stamp> </time_stamp> |
| Latitude | O | Latitude of the data box corner |  | <latitude> </latitude> |
| Longitude | O | Longitude of the data box corner |  | <longitude> </longitude> |
| Altitude | O | Altitude of the data box corner |  | <altitude> </altitude> |

## Document Upload Message

Table B-10 Document Upload Message

| Field Name | Required / Optional | Description |
| --- | --- | --- |
| Submitted by | R | Name of person filing the problem report |
| Date | R | Date submitted (automatic) |
| Title | R | Title of the document |
| Keywords | O | List of keywords to facilitate searching |
| Category | O | Category of document |
| Description | O | Detailed text description of the document |
| Filename | O | Name of file to be uploaded |
| Reference | O | Bibliographic reference to published article |

## Change Management Messages

Table B-11 Problem Report

| Field Name | Required / Optional | Description |
| --- | --- | --- |
| Submitted by | R | Name of person filing the problem report |
| Date | R | Date submitted (automatic) |
| Email Address | O | Email address where problem status reports should be sent |
| Phone | O | Phone number of person submitting the problem report |
| Data Product Name | O | Name of the offending Data Product |
| Problem URL | O | URL of the offending interface page |
| Problem Description | R | Detailed text description of the problem |

Table B-12 Change Request Form

| Field Name | Required / Optional | Description |
| --- | --- | --- |
| Submitted by | R | Name of person filing the change request / problem report |
| Submission date | R | Date submitted (automatic) |
| Email address | R | Email address where change request / problem report status reports should be sent |
| Phone | O | Phone number of person submitting the change request / problem report |
| Problem report | O (R) | If this change is in response to a problem report, its number becomes required and goes here. |
| Item to be changed | R | Data product specification or requirement to be changed |
| Description of change | R | Detailed description of the change, including references as needed. |
| Justification for change | R | Reason that this change should be made |
| Proposed date of change | O | Date when this change should go into effect |
| Impact of change | O | Approximate cost / resource impact of the change |

Table B-13 Change Notification Form

| Field Name | Required / Optional | Description |
| --- | --- | --- |
| Submitted by | R | Name of person filing the change notification / problem report |
| Submission date | R | Date submitted (automatic) |
| Email address | R | Email address where status reports should be sent |
| Phone | O | Phone number of person submitting the change notification / problem report |
| Problem report | O (R) | If this change is in response to a problem report, its number becomes required and goes here. |
| Item to be changed | R | Data product specification or requirement to be changed |
| Description of change | R | Detailed description of the change, including references as needed. |
| Justification for change | R | Reason that this change should be made |
| Proposed date of change | O | Date when this change should go into effect |
| Impact of change | O | Approximate cost / resource impact of the change |

# Data Product Specification

## Data Product Specification

The goal of the Data product Specification is to provide a uniform description of all the data products being exchanged for AIM. There should be a data product specification for each product that is generated and/or exchanged among AIM facilities. These specifications will be assembled and made available as a data location aid, as well as assisting in interface definition.

Table C-14 Data Product Specification

| Field Name | Req / Opt | Notes | Delimiter |
| --- | --- | --- | --- |
| Product Name | R | A unique name for the product | <dp_type> </dp_type> |
| Source | R | The facility generating the product | <dp_source> </dp_source> |
| File Naming Convention | R |  | <name_convention> </name_convention> |
| Description/Purpose | R | Free text describing what the product is for | <description> </description> |
| Data Product Version | R | Required for SDS controlled products only | <dp_version> </dp_version> |
| Data Generation Classifier | R | [routine ¦ analysis ¦ on-demand] | <gen_class> </gen_class> |
| Data Content Classifier | R | [level n ¦ survey ¦ support ¦ education ¦ status ¦ collaborative] | <content_class> </content_class> |
| Generation Trigger | O | Event causing this product to be generated |  |
| Frequency | O | Only required for regularly delivered products | <frequency> </frequency> |
| File Size | O | Approximate upper limit of file size | <file_size> </file_size> |
| File Type | R | [netCDF ¦ ASCII ¦ other] | <file_type> </file_type> |
| Contents | O | ASCII files – description of field contents and formats NetCDF – CDL file for this product Other – format/content definition | <contents> </contents> |
| Intended Recipient | O | See AIM DMP section 7.1 for supported user types | <recipient> </recipient> |
| Comments | O | Free text, any additional descriptions | <comments> </comments> |
| Data Product Version Description | R | Required for SDS products if no reference is available | <dp_version_desc> </dp_version_desc> |
| Product Format Version Description | R | Required for SDS products if no reference is available | <dp_format_desc> </dp_format_desc> |
| Software Major Version Description | R | Required for SDS products if no reference is available | <sw_version_desc> </sw_version_desc> |
| Input/Cal Major Version Description | R | Required for SDS products if no reference is available | <cal_version_desc> </cal_version_desc> |
| DQ-Confidence | R | Confidence level of data product type [High ¦ Reasonable ¦ Low] | <dq_confidence> </dq_confidence> |
| DQ-Version | R | [Current ¦ Preliminary ¦ Supercede ¦ Test ¦ Special] | <dq_version> </dq_version> |
| DQ-Review | R | Level of review product version has undergone [Full ¦ Partial ¦ Inference ¦ None] | <dq_review> </dq_review> |
| DQ-Uses | R | Appropriate Audience for data type [Survey ¦ Quicklook ¦ Detail ¦ Trend ¦ Educational] | <dq_uses> </dq_uses> |
| Parameters | R | List of science parameters contained in this file | <parameters> </parameters> |
| Instrument Mode | R | Instrument modes in effect in this file | <inst_mode> </inst_mode> |
| Max Altitude Covered | R | Maximum value of altitude coverage | <max_alt> </max_alt> |
| Min Altitude Covered | R | Minimum altitude coverage | <min_alt> </min_alt> |
| Reference | O | Reference to another document describing this product | <reference> </reference> |

## Description of Field Value choices

Table C-15 Data Generation Classifier

| Generation Class | Description |
| --- | --- |
| Routine | Produced regularly and systematically with a minimum of human intervention |
| Analysis | - Produced as the result of data analysis or other human intensive process |
| On-demand | - Produced on demand from a user request |

Table C-16 Data Content Classifier

| Content Class | Description |
| --- | --- |
| Raw Telemetry | Unprocessed digital telemetry |
| Level 0 | Unprocessed instrument data at full resolution that has been separated by instrument or subsystem – time ordered with duplication removed |
| Level 1 | Instrument data at full resolution, time-referenced and annotated with ancillary information including geometric parameters, that has been processed to radiometric units |
| Level 2 | Derived geophysical parameters at the retrieval resolution |
| Level 3 | Parameters mapped on a uniform, earth referenced, space-time grid |
| Level 4 | Model output or results from analyses of lower level data (e.g., parameters derived from multiple measurements) |
| Survey | Summary or low fidelity data, often graphically presented, used for quicklook or data location |
| Support | Data acquired from non-AIM sources to supplement data analysis (e.g., NMC data) |
| Collaborative | Data acquired through collaborative sources |
| Educational | Data products and other information intended for use by K-14 educators and students |
| Status | - Data products that contain information about the AIM spacecraft or data products |

Table C-17 Contents

| File Type | Description |
| --- | --- |
| ASCII | Description of field contents and formats. |
| NetCDF | CDL file for this product. |
| Other | Format and content description. |

Table C-18 Data Quality Confidence

| DQ-Confidence | Description |
| --- | --- |
| High | The investigator feels this is an excellent measurement with little or no spurious artifacts. |
| Reasonable | The investigator feels this is a good measurement; however it should be handled with the same care as other remote sensing data sets. It is recommended you consult with the investigator before using this data set. |
| Low | There may be important problems with this data set. Contact the investigator before using this data. |

Table C-19 Data Quality Version

| DQ-Version | Description |
| --- | --- |
| Current | This version represents the best data currently available. |
| Preliminary | This version has been verified and is intended to become the current version. |
| Supercede | This version has been superceded by a more recent version. |
| Test | This version is new and not completely verified. |
| Special | This version has limited applicability. |

Table C-20 Data Quality Review

| DQ-Review | Description |
| --- | --- |
| Full | The data set has been examined and validated. Its overall quality and contents are well known. |
| Partial | The data set has had a cursory examination and has been partially validated. Its overall quality and contents are know, but a full review has not been conducted. |
| Inference | The data set had been validated by a process that has been demonstrated to produce data of a know quality. No review of this individual product has been done. |
| None | The data set has not been reviewed and its quality is unknown. |

Table C-21 Data Quality Uses

| DQ-Uses | Description |
| --- | --- |
| Survey | Appropriate for data location and qualitative purpose only |
| Quicklook | Appropriate for exploratory analysis but not intended to be used for high precision analyses. |
| Detail | Appropriate for most forms of quantitative analysis. |
| Trend | Appropriate for tracking geophysical trends. The data set generally covers long periods of time and will be processed from a large set of quantitatively accurate data sets with stable calibration. |
| Educational | Appropriate for use by K-14 educators and students. |

# Acronyms and Definitions

| A |  |
| --- | --- |
| AIM | Aeronomy of Ice in the Mesosphere |
| C |  |
| CDE | Cosmic Dust Experiment |
| CIPS |  |
| CO-I | Co-Investigator |
| CPU | Central Processing Unit |
| CU | University of Colorado |
| D |  |
| DMP | Data Management Plan |
| DPC | Data Processing Center |
| E |  |
| E/PO | Education and Public Outreach |
| EOS | Earth Observing System |
| F |  |
| FOV | Field-of-View |
| FTP | File Transfer Protocol |
| FTS | Fourier Transform Spectroscopy |
| G |  |
| GATS | G&A Technical Software |
| GIF | Graphics Interchange Format |
| GSE | Ground Support Equipment |
| GSFC | Goddard Space Flight Center |
| GW | Gravity Waves |
| H |  |
| HTTP | Hypertext Transport Protocol |
| HU | Hampton University |
| I |  |
| I&T | Integration and Test |
| IR | Infrared |
| ISO | International Standards Organization |
| ICD | Interface Control Document |
| J |  |
| JPEG | Joint Photographic Experts Group |
| L |  |
| LASP | Laboratory for Atmospheric and Space Physics |
| M |  |
| MOC | Mission Operations Center |
| MODS | Mission Operations Data Systems |
| MPEG | Moving Pictures Expert Group |
| N |  |
| NASA | National Aeronautics and Space Administration |
| NetCDF | Network Common Data Format |
| NSSDC | National Space Sciences Data Center |
| O |  |
| OASIS | Operations and Science Instrument Support |
| OSS | Office of Space Science |
| P |  |
| PA | Product Assurance |
| PDC | Project Data Center |
| PI | Principal Investigator |
| PM | Program Manager |
| PMC | Polar Mesospheric Clouds |
| PNG | Portable Network Graphics |
| POC-DPC | Payload Operations Center and Data Processing Center |
| Q |  |
| QA | Quality Assurance |
| S |  |
| SADIE | Stereoscopic and Dynamics Imaging Instrument |
| S/C | Spacecraft |
| SDL | Space Dynamics Laboratory |
| SDS | Science Data Systems |
| SMTP | Simple Mail Transport Protocol |
| SOFIE | Solar Occultation for Ice Experiment |
| SSH | Secure Shell |
| SSL | Secure Socket Layer |
| SSR | Solid State Recorder |
| STDN | Space Tracking and Data Network |
| STRV | Space Technology Research Vehicle |
| SSU | Stratospheric Sounding Unit |
| T |  |
| TIMED | Thermosphere, Ionosphere, Mesosphere, Energetics and Dynamics |
| U |  |
| UV | Ultraviolet |